Formula 710 Multi-Surface Acid Cleaner

MATERIAL SAFETY DATA SHEET

Tennant

701 North lilac Drive

P.O. Box 1452

Minneapolis, MN 55440-1452

This Material Safety Data Sheet (MSDS) has been prepaired in compliance with Federal OSHA

Hazard Communication Standard 29 CFR 1910, 1200. The information and recommendations

contained herin are based upon data beleived to be correct. However, no guarantee or warranty

of any kind expressed or implied is made with respect to the information contaied herin.

| Emergency Phone Numbers: |  |
| --- | --- |
| CHEMTREC (800) 424-930 (24 Hrs) | District of Columbia (202) 483-7616 (24 hrs) |
| Canada CANUTEC (613) 996-6666 (24hrs) | MSDS Requests: (800) 553-8033 |

| Preparation Info: | By:Judy Wagner |
| --- | --- |
|  | (763) 540-1556 (days) |
|  | (763) 767-8875 (nights) |

Issue Date: 8/10/00; Supercedes: 3/23/00; Printed: 8/16/00.

HAZARD RATING; NFPA; HMIS. Health; 3; 2. Fire; 1; 1. Reactivity; 0; 0. Special; COR; X.

4 = Extreme; 3 = High; 2 = Moderate. 1 = Slight; 0 = Insignificant; COR = Corrosive. X = Ask supervisor for guidance.

**** I. IDENTIFICATION ****

| Product Name: |  | US Dept of Transporation(Ground): |  |  |
| --- | --- | --- | --- | --- |
| Formula 710 Multi-Surface Acid Cleaner |  | UN#: UN1760 |  | Packing Group: II |
| Part Number: | 609801, 609803 | Hazard Classification: |  | Corrosive |
| Chemical Family: | Acid Cleaner | Shipping Name: |  | Corrosive Liquid, N.O.S. |
| Material Use: | Restroom Cleaner | (contains Phosphoric and Sulfmic Acid) |  |  |

**** II. PHYSICAL DATA ****

| Boiling Point: | 212ºF / 100ºC | Freeze Point: | 0ºF / -18ºC |
| --- | --- | --- | --- |
| Density: | 1.1kg/L | % Volatile: | 73.5 by wt. |
| Solubility in Water: | Complete | Vapor Pressure: | N.Av. mm Hg at 20º C. |
| Odor Threshold: | N.AV. | pH: | 1.7 @ 10% |
| Physical Form: | Liquid | Vapor Density: | N.Av. than Air |
| Appearance and Odor | Pink/Wintergreen | Coefficient Water/Oil Distribution: | N.Av. |
| Evaporation Rate: | Slower | VOC g/L (lb/gal): | 402 (3.35) |
| Mechanical Impact: | N.Av. | Static Discharge: | N.Av. |

**** III. INGREDIENTS ****

| CHEMICAL | CAS #: | %W | OSHA PEL ppm | ACGHI TVL TWA ppm | ACGIH TVL STEL ppm | LD50 | LC50 INHALATION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Citric Acid | 77-92-9 | 10-20 | N.Av. | N.Av. | N.Av. | N.Av. | N.Av. |
| Dipropylene Glycol Monomethyl | 34590-94-8 | 10-20 | N.Av. | N.Av. | N.Av. | N.Av. | N.Av. |
| Ether (HDZ) |  |  |  |  |  |  |  |
| Phosphoric Acid (HZD) | 7664-38-2 | 5.3 | N.Av. | 0.25 | 0.75 | N.Av. | N.Av. |
| Sulfamic Acid (HDZ) | 5329-14-6 | 5-10 | N.Av. | N.Av. | N.Av. | N.Av. | N.Av. |
| Surfactant | * | 5-10 | N.Av. | N.Av. | N.Av. | N.Av. | N.Av. |

SARA Title III Section 313 Information:

Components presented in this product at a level which could require reporting under this statute are:

Phosphoric Acid: CAS#:7664-38-2    5.3%by wt.

California Proposition 65 Information:

This product contains the following chemicals that have been designated as cancer and/or reporductive

hazard under California Proposition 65.

Trace amounts of Ethylene Oxide - CAS #75-21-8

Propylene Oxide - CAS #75-56-9 and

1, 4 Diozane CAS #123-91-1

**** IV. FIRE AND EXPLOSIVE HAZARD DATA ****

FLAMMABLE:; No; FLASH POINT:; 170ºF / 76ºC. FLAMMABLE LIMITS; %LEL: N.Av.; % ULE: N.Av.. AUTOIGNITION TEMPERATURE; N.Av..

EXTINGUISHING MEDIA:; use National Fire Protection Association (N.F.P.A.) Class B extinguishers (carbon dioxide, dry chemical, foam).

SPECIAL FIRE FIGHTING PROCEDURES:; Water may be ineffective, but may be used to cool closed containers to prevent pressure buildup possible auto ignition when exposed to extreme heat (fod nozzles prefered). Wear self-contained breathing aparatus.. UNUSUAL FIRE EXPLOSION HAZARD:; Keep containers tightly closed from heat, electrical equipment, sparks and open flame. Closed containers may explode when exposed to extreme heat. Do not apply to hot surfaces..

**** V. REACTIVITY DATA ****

| STABILITY: | Stable | HAZARDOUS POLYMERIZATION: |  | Will Not Occur |
| --- | --- | --- | --- | --- |
| CONDITIONS TO AVOID: |  |  | N.Av. |  |
| INCOMPATIBILITY (materials to avoid): |  |  | Strong Alkali. |  |
| HAZARDOUS DECOMPOSITION PRODUCTS: |  |  | Carbon dioxide, carbon monoxide, and oxides phosphorus, nitrogen, sulfer, ammonia, or amines. |  |

**** VI. HEALTH HAZARD DATA ****

| EFFECTS OF OVEREXPOSURE: | THRESHOLD LIMIT VALUE - See Section III for individual ingredients |
| --- | --- |
| PRIMARY ROUTES OF ENTRY: | Inhalation, Skin Contact |
| SKIN CONTACT/ABSORPTION: | Skin contact may cause severe irritation. Prolonged or repeated skin contact may cause burns. |
| EYE CONTACT: | Extremely irritatingwith possible permanent eye injury. |
| INGESTION (swallowing): | Contains materials which will cfause a burning sensation, gastric upset, diarrhea and severe inflamation to the mucous membranes. |
| INHALATION (breathing): | Slight headache. May burn nasal passages. |
| SUPPLEMENTAL HEALTH INFORMATION: |  |
| Reports have associated repeated and prolonged overexposure to solvents with permanent brain and nervous system damage. Intentinal misuse by deliberately concentrating this product may be harmful or fatal. |  |

**** VII. EMERGENCY AND FIRST AID PROCEDURES ****

| SKIN CONTACT: | Wash immediately with mild soap and water. Rinse thoroughly with cool water. |
| --- | --- |
| EYE CONTACT: | Flush immediately with water for 15 minutes: CALL A PHYSICIAN. |
| INGESTION (swallowing): | CALL A PHYSICIAN OR POISION CONTOL CENTER IMMEDIATELY. DO NOT attempt to induce vomiting. give two glasses of water and have this document available. |
| INHALATION (breathing): | Remove to fresh air. |

**** VIII. PREVENTATIVE MEASURES ****

SPILL OR LEAK PROCEDURE:

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: Absorb with inert material (sand, vermiculite, etc.). Sweep or scoop ua and dispose container. Flush area with water; prevent washings from entering waterways. WASTE DISPOSAL METHOD Dispose of in accordance with federal, state and local regualtions.

SPECIAL PROTECTION INFORMATION: RESPIRATORY PROTECTION (specify type) and VENTILATION (local exhaust): Unless air monitoring demonstrates vapor/mist levels above applicable limits, no respiratory is required. If respirator is required, the appropriate, properly fitted respirator (NIOSH/MSHA approved) should be worn during application. Follow respirator manufacturors directions for respirator use. PROTECTIVE GLOVES (specify type): Impervious gloves; i.e, rubber, nitrile EYE PROTECTION (specify type): Chemical splash goggles

SPECIAL PERCAUTIONS: PERCAUTIONS TO BE TAKEN IN HANDELING AND STORAGE: Store in a cool (below 100ºF, 37.5ºC), dry place. OTHER PERCATUIONS: Do not take internally. Avoid prolonged contact with skin; contaminated clothing should be removed immediately. Avoid free fall of liquid in excess of a few inches.