| Material Safety Data Sheet May be used to comply with OSHA's Hazard Communication Standard, 29CFR 1910. 1200. Standard must be consulted for specific requirements. | NEVR-DULL MAGIC WADDING POLISH QUICK INDENTIFIER Common Name: (used on label and list) TRADE SECRET (Proprietary) |
| --- | --- |

| SECTION 1 - |  |  |  |
| --- | --- | --- | --- |
| Manufacturer's Name | THE GEORGE BASCH CO., INC. |  |  |
| Address | 19 HANSE AVE - P.O. BOX 188 | Emergency Telephone No. | 516-378-8100 |
| City, State, and ZIP | FREEPORT, N.Y. 11520 | Other Information Calls | same |
| Signature of Person Responsible for Preparation (Optional) |  | Date Prepared | 1-2-2008 |

| SECTION 2 - HAZARDOUS INGREDIENTS/IDENTITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazardous Component(s) (chemical & common name(s)) | OSHA PEL | ACGIII TLV | Other Exposure Limits | % (optional) | CAS NO. |  |
| Mineral Spirits – Petroleum Hydrocarbon |  |  |  |  |  |  |
| This product is C.A.R.B. , V.O.C. Compliant |  |  |  |  |  |  |

| SECTION 3 - PHYSICAL & CHEMICAL CHARACTERISTICS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Boiling Point | 343 F |  |  | Specific Gravity (H2O = 1) | 0.805 @60 F | Vapor Pressure (mm Hg)@20c | 0.6 |
|  |  | Vapor Density (Air = 1) | 5.8 |  |  |  |  |
| Solubility in Water | Stable under normal conditions |  |  | Reactivity in Water |  | None |  |
| Appearance and Odor | cotton wadding | Impregnated with liquid chemical |  | Melting Point |  | NA |  |

| SECTION 4 - FIRE & EXPLOSION DATA |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flash Point | 145 F. 63 C. | Method Used | Open cup | Flammable Limits in Air % by Volume | LEL Lower | 0.9 | UEL Upper | NA |
| Auto-Ignition Temperature | 490 F | Extinguisher Media | Fire Extinguisher- type-Monoamnonium Phosphate (class ABC - Dry Chemical) |  |  |  |  |  |
| Special Fire Fighting Procedures |  | Keep away from open flame- |  |  |  |  |  |  |
| Unusual Fire and Explosion Hazards |  | None |  |  |  |  |  |  |

| SECTION 5 - PHYSICAL HAZARDS (REACTIVITY DATA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stability | Unstable [ ] Stable [x] | Conditions to Avoid | Isolate from strong oxidizers, heat and open flame |  |  |
| Incompatibility (Materials to Avoid) | Isolate from strong oxidizers such as permanganates, chromates & peroxides |  |  |  |  |
| Hazardous Decomposition Products | Carbon monoxide, Carbon Dioxide from burning |  |  |  |  |
| Hazardous Polymerization | Will Not Occur |  |  |  |  |

| SECTION 6 - HEALTH HAZARDS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Eye Contact | Slightly irritating but does not injure eye tissue |  |  |  |  |  |  |
| Skin Contact Low order of toxicity. Skin contact may irritate or cause an existing dermatitis condition |  |  |  |  |  |  |  |
| Inhalation | Low vapor concentrations may cause dizziness, headaches, drowsiness |  |  |  |  |  |  |
| Ingestion | May cause local irritation, vomiting, respiratory irritation, headache, dizziness, nausea |  |  |  |  |  |  |

| SECTION 7 – First Aid Measures |  |
| --- | --- |
| Eye Contact | Flush eyes with large amounts of water until irritation subsides |
| Skin Contact | Recommend rubber gloves to use. Wash hands with soap and water |
| Inhalation | Remove to fresh air. Give artificial respiration if necessary |
| Ingestion | If swallowed DO NOT induce vomiting. Seek medical attention |

| SECTION 8 – PRECAUTIONS FOR SAFE HANDLING AND USE |  |  |
| --- | --- | --- |
| Precautions to be Taken in Handling and Storage | Store at room temperature. Keep away from open flames |  |

SECTION 9 – STABILITY AND REACTIVITY

| Stability | Stable | Conditions to Avoid Instability | N/A | Hazardous Polymerization | Will Not Occur |
| --- | --- | --- | --- | --- | --- |
| Conditions to Avoid Hazardous Polymerization | N/A | Materials & Conditions to avoid Incompatibility | Oxidizers | Hazardous Decomposition Products | None |

SECTION 10 – HAZARD RATING SYSTEM

|  | NPCA-HMIS | NFPA 704 | KEY 4 = Severe 3 = Serious 2 = Moderate 1 = Slight 0 = Minimal |
| --- | --- | --- | --- |
| Nat’l Paint & Coating Assoc./ Hazardous indent. Systems |  | National Fire Protection Association |  |
| HEALTH | 1 | 0 |  |
| FLAMMABILITY | 2 | 2 |  |
| REACTIVITY | 0 | 0 |  |