SAFETY DATA SHEET

This Safety Data Sheet conforms to ANSI Z400.5, and to the format requirements

and the International Chemical Safety Cards of the Global Harmonizing System.

THIS SDS COMPLIES WITH 29 CFR 1910.1200 (HAZARD COMMUNICATION STANDARD)

IMPORTANT: Read this SDS before handling & disposing of this product.

Pass this information on to employees, customers, & users of this product.

SECTION 1. IDENTIFICATION OF THE SUBSTANCE OR MIXTURE AND OF THE SUPPLIER

PRODUCT IDENTITY: GEL BREAK CS05

PRODUCT PURPOSE: SPECIAL ADHESIVE REMOVER

COMPANY IDENTITY: Bridgepoint Systems

COMPANY ADDRESS:  4282 S 590 W

COMPANY CITY:     Salt Lake City, UT 84123

COMPANY PHONE:    1-800-658-5314

EMERGENCY PHONES: INFOTRAC: 1-800-535-5053 (USA)

SECTION 2. HAZARDS IDENTIFICATION

WARNING!

EXPOSURE PREVENTION: STRICT HYGIENE!

HAZARD STATEMENTS:

H100s = General, H200s = Physical, H300s = Health, H400s = Environmental

H226         Combustible liquid (North America), Flammable liquid & vapor (Elsewhere).

H302         Harmful if swallowed.

H315         Causes skin irritation.

H317         May cause an allergic skin reaction.

H320         Causes eye irritation.

H335         May cause respiratory irritation.

PRECAUTIONARY STATEMENTS:

P100s = General, P200s = Prevention, P300s = Response, P400s = Storage, P500s = Disposal

P243         Take precautionary measures against static discharge.

P273         Avoid release to the environment.

P280         Wear protective gloves/protective clothing/eye protection/face protection.

P301+330+331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P305+351+338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact

lenses if present & easy to do - Continue rinsing.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

MATERIAL                                CAS#          EINECS#       WT %

d-Limonene                        5989-27-5        227-813-5   Trade Secret

2(2-Butoxyethoxy)ethanol           112-34-5        203-961-6   Trade Secret

Methoxypropoxypropanol           34590-94-8        252-104-2   Trade Secret

Orange Peel Terpene               8028-48-6            -       Trade Secret

Trace components: Trace ingredients (if any) are present in < 1% concentration,

(< 0.1% for potential carcinogens, reproductive toxins, respiratory tract mutagens,

and sensitizers). None of the trace ingredients contribute significant additional

hazards at the concentrations that may be present in this product. All pertinent hazard

information has been provided in this document, per the requirements of the Federal

Occupational Safety and Health Administration Standard (29 CFR 1910.1200), U.S. State

equivalents, and Canadian Hazardous Materials Identification System Standard (CPR 4).

SEE SECTIONS 8, 11 & 12 FOR TOXICOLOGICAL INFORMATION.

SECTION 4. FIRST AID MEASURES

GENERAL ADVICE:

First Aid responders should pay attention to self-protection and use the recommended

protective clothing (chemical resistant gloves, splash protection). If potential for

exposure exists, refer to Section 8 for specific personal protective equipment.

COMPANY IDENTITY:  Bridgepoint Systems                              SDS DATE: 05/12/2013

PRODUCT IDENTITY:  GEL BREAK                                        ORIGINAL: 05/12/2013

SECTION 4. FIRST AID MEASURES (CONTINUED)

EYE CONTACT:

If this product enters the eyes, open eyes while under gently running water. Use

sufficient force to open eyelids. "Roll" eyes to expose more surface. Minimum flushing

is for 15 minutes. Seek immediate medical attention.

SKIN CONTACT:

If the product contaminates the skin, immediately begin decontamination with running

water. Minimum flushing is for 15 minutes. Remove contaminated clothing, taking care not

to contaminate eyes. If skin becomes irritated and irritation persists, medical attention

may be necessary. Wash contaminated clothing before reuse, discard contaminated shoes.

INHALATION:

After high vapor exposure, remove to fresh air. If it is suspected that fumes are still

present, the rescuer should wear an appropriate mask or self-contained breathing

apparatus. Keep person warm and at rest. breathing is difficult, give

oxygen.  If breathing has stopped, trained personnel should immediately begin artificial

respiration. It may be dangerous to the person providing aid to give mouth-to-mouth

resuscitation. If unconscious, place in recovery position and get medical attention

immediately. Maintain an open airway. Loosen tight clothing such as a collar, tie, belt

or waistband. If the heart has stopped, trained personnel should immediately begin

cardiopulmonary resuscitation (CPR). Seek immediate medical attention. In case of

inhalation of decomposition products in a fire, symptoms may be delayed.The exposed person

may need to be kept under medical surveillance for 48 hours.

SWALLOWING:

If swallowed, CALL PHYSICIAN OR POISON CONTROL CENTER FOR MOST CURRENT INFORMATION. If

professional advice is not available, give two glasses of water to drink. DO NOT INDUCE

VOMITING. Never induce vomiting or give liquids to someone who is unconscious, having

convulsions, or unable to swallow. Seek immediate medical attention.

NOTES TO PHYSICIAN:

There is no specific antidote. Treatment of overexposure should be directed at the control

of symptoms and the clinical condition of the patient. Any material aspirated during

vomiting may cause lung injury. Therefore, emesis should not be induced mechanically or

pharmacologically. If it is considered necessary to evacuate the stomach contents, this

should be done by means least likely to cause aspiration (such as: Gastric lavage after

endotracheal intubation).

SECTION 5. FIRE FIGHTING MEASURES

FIRE & EXPLOSION PREVENTIVE MEASURES

NO open flames, NO sparks, & NO smoking. Above flash point, use

a closed system, ventilation, explosion-proof electrical equipment, lighting.

EXTINGUISHING MEDIA

Use dry powder, AFFF, alcohol-resistant foam, water spray, carbon dioxide.

SPECIAL FIRE FIGHTING PROCEDURES

Water spray may be ineffective on fire but can protect fire-fighters

& cool closed containers. Use fog nozzles if water is used.

Do not enter confined fire-space without full bunker gear.

(Helmet with face shield, bunker coats, gloves & rubber boots).

Use NIOSH approved positive-pressure self-contained breathing apparatus.

UNUSUAL EXPLOSION AND FIRE PROCEDURES

COMBUSTIBLE!

Isolate from oxidizers, heat, & open flame.

Closed containers may explode if exposed to extreme heat.

Applying to hot surfaces requires special precautions.

Empty container very hazardous! Continue all label precautions!

COMPANY IDENTITY:  Bridgepoint Systems                              SDS DATE: 05/12/2013

PRODUCT IDENTITY:  GEL BREAK                                        ORIGINAL: 05/12/2013

SECTION 6. ACCIDENTAL RELEASE MEASURES

SPILL AND LEAK RESPONSE AND ENVIRONMENTAL PRECAUTIONS:

Uncontrolled releases should be responded to by trained personnel using pre-planned

procedures. Proper protective equipment should be used. In case of a spill, clear the

affected area, protect people, and respond with trained personnel.

PERSONAL PROTECTIVE EQUIPMENT

The proper personal protective equipment for incidental releases (such as: 1 Liter of the

product released in a well-ventilated area), use impermeable gloves (triple-gloves (rubber

gloves and nitrile gloves, over latex gloves), goggles, face shield, and appropriate body

protection. In the event of a large release, use impermeable gloves, specific for the

material handled, chemically resistant suit and boots, and hard hat. Self-Contained

Breathing Apparatus or respirator may be required where engineering controls are not

adequate or conditions for potential exposure exist. When respirators are required, select

NIOSH/MSHA approved based on actual or potential airborne concentrations in accordance

with latest OSHA and/or ANSI recommendations.

ENVIRONMENTAL PRECAUTIONS:

Stop spill at source. Construct temporary dikes of dirt, sand, or any appropriate readily

available material to prevent spreading of the material. Close or cap valves and/or

block or plug hole in leaking container and transfer to another container. Keep from

entering storm sewers and ditches which lead to waterways, and if necessary, call the

local fire or police department for immediate emergency assistance.

CONTAINMENT AND CLEAN-UP MEASURES:

Absorb spilled liquid with polypads or other suitable absorbent materials. If necessary,

neutralize using suitable buffering material, (acid with soda ash or base with phosphoric

acid), and test area with litmus paper to confirm neutralization. Clean up with

non-combustible absorbent (such as: sand, soil, and so on). Shovel up and place all spill

residue in suitable containers. dispose of at an appropriate waste disposal facility

according to current applicable laws and regulations and product characteristics at time

of disposal (see Section 13 - Disposal Considerations).

SECTION 7. HANDLING AND STORAGE

HANDLING

Isolate from oxidizers, heat, & open flame. Use only with adequate ventilation. Avoid or

repeated breathing of vapor or spray mist. Do not get in eyes, on skin or clothing.

Wear OSHA Standard goggles or face shield. Consult Safety Equipment Supplier. Wear

goggles, face shield, gloves, apron & footwear impervious to material. Wash clothing

before reuse. Avoid free fall of liquid. Ground containers when transferring. Do not flame

cut, braze, or weld. Empty container very hazardous! Continue all label precautions!

STORAGE

Keep in fireproof surroundings. Keep separated from strong oxidants. Store in an area

without a drain or sewer access. Do not store above 49 C/120 F.

Keep container tightly closed & upright when not in use to prevent leakage.

NONBULK: CONTAINERS:

Store containers in a cool, dry location, away from direct sunlight, sources of intense

heat, or where freezing is possible. Material should be stored in secondary containers or

in a diked area, as appropriate. Store containers away from incompatible chemicals (see

Section 10, Stability and Reactivity). Post warning and "NO SMOKING" signs in storage and

use areas, as appropriate. Empty containers should be handled with care. Never store food,

feed, or drinking water in containers which held this product.

PROTECTIVE PRACTICES DURING MAINTENANCE OF CONTAMINATED EQUIPMENT:

Follow practices indicated in Section 6 (Accidental Release Measures).

Always use this product in areas where adequate ventilation is provided. Collect

all rinsates and dispose of according to applicable Federal, State, Provincial,

or local procedures.

COMPANY IDENTITY:  Bridgepoint Systems                              SDS DATE: 05/12/2013

PRODUCT IDENTITY:  GEL BREAK                                        ORIGINAL: 05/12/2013

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

MATERIAL                                CAS#     EINECS#  TWA (OSHA)     TLV (ACGIH)

d-Limonene                        5989-27-5   227-813-5  None Known     None Known

2(2-Butoxyethoxy)ethanol           112-34-5   203-961-6  None Known       25 ppm

Methoxypropoxypropanol           34590-94-8   252-104-2   100 ppm        100 ppm

Orange Peel Terpene               8028-48-6       -      None Known       50 ppm

MATERIAL                        CAS#     EINECS#   CEILING  STEL(OSHA/ACGIH)  HAP

2(2-Butoxyethoxy)ethanol     112-34-5  203-961-6  None Known   None Known    Yes

Each component showing `Yes' under "HAP" is an EPA Hazardous Air Pollutant.

RESPIRATORY EXPOSURE CONTROLS

Seek professional advice prior to respirator selection and use.

Maintain airborne contaminant concentrations below exposure limits given above. If

respiratory protection is needed, use only protection authorized in 29 CFR 1910.134,

European Standard EN 149, or applicable State regulations. If adequate ventilation is

not available or there is potential for airborne exposure above the exposure limits, a

respirator may be worn up to the respirator exposure limitations, check with respirator

equipment manufacturer's recommendations/limitations. For a higher level of protection,

use positive pressure supplied air respiration protection or Self-Contained Breathing

Apparatus or if oxygen levels are below 19.5% or are unknown.

EMERGENCY OR PLANNED ENTRY INTO UNKNOWN CONCENTRATIONS OR IDLH CONDITIONS

Positive pressure, full-face piece Self-Contained Breathing Apparatus; or positive

pressure, full-face piece Self-Contained Breathing Apparatus with an auxilliary positive

pressure Self-Contained Breathing Apparatus.

VENTILATION

LOCAL EXHAUST:   Necessary          MECHANICAL (GENERAL):  Necessary

SPECIAL:         None               OTHER:                 None

Please refer to ACGIH document, "Industrial Ventilation, A Manual of

Recommended Practices", most recent edition, for details.

EYE PROTECTION:

Splash goggles or safety glasses.

HAND PROTECTION:

Wear appropriate impervious gloves for routine industrial use. Use impervious gloves for

spill response, as stated in Section 6 of this SDS (Accidental Release Measures).

NOTICE: The selection of a specific glove for a particular application and duration of use

in a workplace should also take into account all relevant workplace factors such as, but

not limited to: Other chemicals which may be handled, physical requirements (cut/puncture

protection, dexterity, thermal protection), potential body reactions to glove materials,

as well as the instructions/specifications provided by the glove supplier.

SECTION 9. PHYSICAL & CHEMICAL PROPERTIES

APPEARANCE:                                       Viscous Cloudy Liquid,

ODOR:                                             Citrus

ODOR THRESHOLD:                                   Not Available

pH (Neutrality):                                  Not Applicable

MELTING POINT/FREEZING POINT:                     Not Available

BOILING RANGE (IBP,50%,Dry Point):                111 160 226*C/232 320 440*F(*=End Point)

FLASH POINT (TEST METHOD):                        39 C / 102 F (TCC) (Lowest Component)

EVAPORATION RATE (n-BUTYL ACETATE=1):             Not Applicable

FLAMMABILITY CLASSIFICATION:                      Class II

LOWER FLAMMABLE LIMIT IN AIR (% by vol):          0.95 (Lowest Component)

UPPER FLAMMABLE LIMIT IN AIR (% by vol):          Not Available

VAPOR PRESSURE (mm of Hg)@20 C                    5.4

VAPOR DENSITY (air=1):                            3.9

COMPANY IDENTITY:  Bridgepoint Systems                              SDS DATE: 05/12/2013

PRODUCT IDENTITY:  GEL BREAK                                        ORIGINAL: 05/12/2013

SECTION 9. PHYSICAL & CHEMICAL PROPERTIES (CONTINUED)

GRAVITY @ 68/68 F / 20/20 C:

DENSITY:                                       0.880

SPECIFIC GRAVITY (Water=1):                    0.881

POUNDS/GALLON:                                 7.335

WATER SOLUBILITY:                                 Appreciable

PARTITION COEFFICIENT (n-Octane/Water):           Not Available

AUTO IGNITION TEMPERATURE:                        232 C /  450 F

DECOMPOSITION TEMPERATURE:                        Not Available

REFRACTIVE INDEX:                                 1.446

MIXED ANILINE POINT (Acid Insol):                 48 C / 119 F

VOCs (>0.044 Lbs/Sq In) :                         Not Available

TOTAL VOC'S (TVOC)*:                              4.05 Lbs/Gal

NONEXEMPT VOC'S (CVOC)*:                          .1 Lbs/Gal

HAZARDOUS AIR POLLUTANTS (HAPS):                  28.5  Wt%

NONEXEMPT VOC PARTIAL PRESSURE (mm of Hg @ 20 C)  Not Available

VISCOSITY @ 25 C (ASTM E2196):                    3479 cPs

* Using California Air Resources Board (CARB) Rule 310.

SECTION 10. STABILITY & REACTIVITY

STABILITY

Stable under normal conditions.

CONDITIONS TO AVOID

Isolate from oxidizers, heat, & open flame.

MATERIALS TO AVOID

Reacts with strong oxidants, causing fire & explosion hazard.

HAZARDOUS DECOMPOSITION PRODUCTS

Carbon Monoxide, Carbon Dioxide from burning.

HAZARDOUS POLYMERIZATION

Will not occur.

SECTION 11. TOXICOLOGICAL INFORMATION

ACUTE HAZARDS

EYE & SKIN CONTACT:

Primary irritation to skin, defatting, dermatitis.

Absorption thru skin increases exposure.

Primary irritation to eyes, redness, tearing, blurred vision.

Liquid can cause eye irritation. Wash thoroughly after handling.

INHALATION:

Anesthetic. Irritates respiratory tract. Acute overexposure

can cause serious nervous system depression. Vapor harmful.

SWALLOWING:

Harmful or fatal if swallowed.

Swallowing can cause abdominal irritation, nausea, vomiting & diarrhea.

The symptoms of chemical pneumonitis may not show up for a few days.

SUBCHRONIC HAZARDS/CONDITIONS AGGRAVATED

CONDITIONS AGGRAVATED

Persons with severe skin, liver or kidney problems should avoid use.

CHRONIC HAZARDS

CANCER, REPRODUCTIVE & OTHER CHRONIC HAZARDS:

This product has no carcinogens listed by IARC, NTP, NIOSH,

OSHA or ACGIH, as of this date, greater or equal to 0.1%.

Absorption thru skin may be harmful.

COMPANY IDENTITY:  Bridgepoint Systems                              SDS DATE: 05/12/2013

PRODUCT IDENTITY:  GEL BREAK                                        ORIGINAL: 05/12/2013

SECTION 11. TOXICOLOGICAL INFORMATION (CONTINUED)

IRRITANCY OF PRODUCT: This product is irritating to contaminated tissue.

SENSITIZATION TO THE PRODUCT: No component of this product is known to be a sensitizer.

MUTAGENICITY: This product is not reported to produce mutagenic effects in humans.

EMBRYOTOXICITY: This product is not reported to produce embryotoxic effects in humans.

TERATOGENICITY: This product is not reported to produce teratogenic effects in humans.

REPRODUCTIVE TOXICITY: This product is not reported to cause reproductive effects in humans.

A mutagen is a chemical which causes permanent changes to genetic material (DNA) such that

the changes will propagate through generational lines. An embryotoxin is a chemical which

causes damage to a developing embryo (such as: within the eight weeks of pregnancy in

humans), but the damage does not propagate across generational lines. A teratogen is a

chemical which causes damage to a developing fetus, but the damage does not propagate

across generational lines. A reproductive toxin is any substance which interferes in any way

with the reproductive process.

MAMMALIAN TOXICITY INFORMATION

MATERIAL                        CAS#     EINECS#     LOWEST KNOWN LETHAL DOSE DATA

LOWEST KNOWN LD50 (ORAL)

Methoxypropoxypropanol     34590-94-8  252-104-2    4900.0 mg/kg(Rats)

LOWEST KNOWN LD50 (SKIN)

2(2-Butoxyethoxy)ethanol     112-34-5  203-961-6    4120.0 mg/kg (Rabbits)

SECTION 12. ECOLOGICAL INFORMATION

ALL WORK PRACTICES MUST BE AIMED AT ELIMINATING ENVIRONMENTAL CONTAMINATION.

EFFECT OF MATERIAL ON PLANTS AND ANIMALS:

This product may be harmful or fatal to plant and animal life if released

into the environment. Refer to Section 11 (Toxicological Information) for

further data on the effects of this product's components on test animals.

EFFECT OF MATERIAL ON AQUATIC LIFE:

The most sensitive known aquatic group to any component of this product is:

Daphnia Magna exposed to 1919 ppm or mg/L are adversely affected by

components of this product. Keep out of sewers and natural water supplies.

MOBILITY IN SOIL

This material is a mobile liquid.

DEGRADABILITY

This product is partially biodegradable.

ACCUMULATION

Bioaccumulation of this product has not been determined.

SECTION 13. DISPOSAL CONSIDERATIONS

Processing, use or contamination may change the waste disposal requirements. Do not

dispose of on land, in surface waters, or in storm drains. Waste should be recycled

or disposed of in accordance with regulations. Large amounts should be collected

for reuse or consigned to licensed hazardous waste haulers for disposal.

ALL DISPOSAL MUST BE IN ACCORDANCE WITH ALL FEDERAL, STATE, PROVINCIAL, AND LOCAL

REGULATIONS. IF IN DOUBT, CONTACT PROPER AGENCIES. EPA CHARACTERISTIC: D001

COMPANY IDENTITY:  Bridgepoint Systems                              SDS DATE: 05/12/2013

PRODUCT IDENTITY:  GEL BREAK                                        ORIGINAL: 05/12/2013

SECTION 14. TRANSPORT INFORMATION

Ground Transportation: Limited quantity exception applies to inner packages of less

than 1.3 gallons. Inner packages containing more than 1.3 gallon, must be transported as

NA1993; Flammable Liquid, NOS (D-Limonene), Comb Liq, III.

Air Transportation: With a vented seal/cap, this item cannot be shipped via air in

accordance with CFR 49 173-24(g). Check regulations for international shipments

restrictions.

Maritime Transportation: Check regulations for international shipments restrictions.

SECTION 15. REGULATORY INFORMATION

EPA REGULATION:

SARA SECTION 311/312 HAZARDS: Acute Health, Fire

All components of this product are on the TSCA list.

SARA Title III Section 313 Supplier Notification

This product contains the indicated <*> toxic chemicals subject to the

reporting requirements of Section 313 of the Emergency Planning & Community

Right-To-Know Act of 1986 & of 40 CFR 372. This information must be

included in all MSDSs that are copied and distributed for this material.

SARA TITLE III INGREDIENTS        CAS#    EINECS#    WT%   (REG.SECTION)   RQ(LBS)

*2(2-Butoxyethoxy)ethanol     112-34-5 203-961-6  20-30  (313)              None

STATE REGULATIONS:

CALIFORNIA SAFE DRINKING WATER & TOXIC ENFORCEMENT ACT (PROPOSITION 65):

This product contains no chemicals known to the State of California

to cause cancer or reproductive toxicity.

INTERNATIONAL REGULATIONS

The components of this product are listed on the chemical inventories of the

following countries:

Australia (AICS), Canada (DSL or  NDSL), China (IECSC), Europe (EINECS, ELINCS)G

Japan (METI/CSCL, MHLW/ISHL), South Korea (KECI), New Zealand (NZIoC),

Philippines (PICCS), Switzerland (SWISS), Taiwan (NECSI), USA (TSCA).

CANADA: WORKPLACE HAZARDOUS MATERIALS INFORMATION SYSTEM (WHMIS)

B3:  Combustible Liquid.

D2B: Irritating to skin / eyes.

This product has been classified in accordance with hazard criteria of the Controlled

Products Regulations (CPR) and the SDS contains all the information required by the CPR.

SECTION 16. OTHER INFORMATION

HAZARD  RATINGS:

HEALTH (NFPA): 1,   HEALTH (HMIS): 1,   FLAMMABILITY: 2,   PHYSICAL HAZARD: 0

(Personal Protection Rating to be supplied by user based on use conditions.)

This information is intended solely for the use of individuals

trained in the NFPA & HMIS hazard rating systems.

EMPLOYEE TRAINING

See Section 2 for Risk & Safety Statements. Employees should be made aware

of all hazards of this material (as stated in this SDS) before handling it.

SDS DATE: 05/12/2013

NOTICE

Bridgepoint Systems disclaims all expressed or implied warranties of merchantability or fitness for a specific use, with respect to the product or the information provided herein,

except for conformation to contracted specifications. All information appearing herein

COMPANY IDENTITY:  Bridgepoint Systems                              SDS DATE: 05/12/2013

PRODUCT IDENTITY:  GEL BREAK                                        ORIGINAL: 05/12/2013

SECTION 16. OTHER INFORMATION

is based upon data obtained from manufacturers and/or recognized technical sources.

While the information is believed to be accurate, we make no representations as to its

accuracy or sufficiency.

Conditions of use are beyond our control, and therefore users are responsible for

verifying the data under their own operating conditions to determine whether the product

is suitable for their particular purposes and they assume all risks of their handling,

and disposal of the product. Users also assume all risks in regards to the publication

or use of, or reliance upon information contained herein.

This information relates only to the product designated herein, and does not relate to

its use in combination with any other material or process.