### MATERIAL SAFETY DATA SHEET

ITEM NUMBER                              2190

BRAND NAME:                              Sanitex

- PRODUCT NAME:		VINYL NON-MEDICAL PRE-POWDEREDGLOVES,

TRADE NAME:			“SANITEX” VINYL GENERAL-PURPOSE GLOVES

FDA 510K NO:			NOT REQUIRED

MATERIAL:			POLYVINYLCHLORIDE (PVC)

DONNING POWDER:		USP ABSORBABLE DUSTING POWDER

D.O.T  RATING:			NOT RESTRICTED

SAMPLING PLAN                          S-4, AQL ABOUT 2.5

Sanitex Gloves are first grade gloves freshly manufactured on the same production line that medical gloves are made but without going through the rigorous FDA medical inspection.  We do not use rejects or 2nds.  Our gloves are made with ingredients in compliance with 21CFR as accepted by the USDA for use in food processing industry.

SPECIFICATION

| SIZE (ITEM #) | Small (2192) | Medium (2193) | Large (2194) | X-Large (2195) |
| --- | --- | --- | --- | --- |
| Length (mm) | 230 | 235 | 240 | 240 |
| Width at Palm (mm) | 85±5 | 95±5 | 105±5 | 115±5 |
| Min. Thickness at Palm mm (mil) | 0.08mm (3.1 mil) | 0.08mm (3.1 mil) | 0.08 (3.1 mil) | 0.08 (3.1 mil) |
| Min. Thickness at Finger mm (mil) | 0.08mm (3.1 mil) | 0.08mm (3.1 mil) | 0.08 (3.1 mil) | 0.08 (3.1 mil) |

PROPERTIES

| BEFORE AGING |  |
| --- | --- |
| TENSILE STRENGTH | MIN 11 MPa |
| ULTIMATE ELONGATION | MIN 300% |
| POWDERED LEVEL (average) | 0.01gram per square dm. |
| ORIENTATION | Ambidextrous |
| CUFF | Rounded Bead |
| COLOR | Natural Clear |

CORN STARCH

| APPREARENCE | PH | MOISTURE | ASH CONTENT | SEDIMENTATION |
| --- | --- | --- | --- | --- |
| White Free Flowing Powder | 10.0 to 10.8 | 12.0% (MAX) | 3.0% (MAX) | 45 to75 ML |

# FIRE AND EXPLOSION

| STABILITY | FLASH POINT | EXTINGUISHING MEDIA | CONDITION TO AVOID | IMCOMPATIBLE MATERIALS |
| --- | --- | --- | --- | --- |
| Stable | N/A | Foam ,Water, Carbon dioxide | Extreme Heat | None |

PACKING	100 pieces per box, 10 dispenser boxes per case

14 cases/layer, 8 layers high, 112 cases/pallet.

Pallet Dimensions: 45” x 48” x 85” high.

STORAGE 	Store at room temperature.  Keep away from heat.

Avoid direct sunlight and fluorescent light.

CAUTION        Components used in making these gloves may cause adverse allergy in some users and

patients.  Immediately discontinue use and seek medical assistance if reactions are suspected.

Gloves are not intended for use as a general chemical barrier. Not for medical use.