MATERIAL SAFETY DATA SHEET

This MSDS complies with OSHA'S Hazard Communication Standard 29 CFR 1910.1200 and OSHA Form 174

| IDENTITY AND MANUFACTURER'S INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NFPA Rating: Health-2; Flammability-1; Reactivity-0; Special-0 |  | HMIS Rating: Health-2; Flammability-1; Reactivity-0; Personal Protection- 0 |  |  |  |  |  |  |
| DISTRIBUTED BY: MISSION LABORATORIES ADDRESS: 2433 BIRKDALE STREET LOS ANGELES, CA 90031 |  | DOT Hazard Classification: NON HAZARDOUS Identify (trade name as used on label): AMAZING BONNET |  |  |  |  |  |  |
| Date Prepared: 04/04/03 Prepared By: MM |  | MSDS Number: F2121 Revision- |  |  |  |  |  |  |
| Information Calls: (626) 333-7040 EMERGENCY RESPONSE NUMBER: (800) 255-3924 |  | NOTICE: JUDGMENT BASED ON INDIRECT TEST DATA |  |  |  |  |  |  |
| SECTION 1 - MATERIAL IDENTIFICATION AND INFORMATION |  |  |  |  |  |  |  |  |
| COMPONENTS-CHEMICAL NAMES AND COMMON NAMES (Hazardous Components 1% or greater; Carcinogens 0.1% or greater) |  |  | CAS Number | SARA III LIST |  | OSHA PEL (ppm) | ACGIH TLV (ppm) | Carcinogen Ref. Source ** |
| PETROLEUM DISTILLATE |  |  | 64742-47-8 | N/E |  | N/E | N/E | N/E |
| SECTION 2 - PHYSICAL/CHEMICAL CHARACTERISTICS |  |  |  |  |  |  |  |  |
| Boiling Point: 212 F |  | Specific Gravity : 0.925 |  |  |  |  |  |  |
| Vapor Pressure: N/E Max. |  | pH of Product : 8.5 +/- 0.5 |  |  |  |  |  |  |
| Vapor Density N/E |  | Evaporation Rate: N/E |  |  |  |  |  |  |
| Solubility in Water: MISCIBLE |  | Water Reactive: N/E |  |  |  |  |  |  |
| Appearance and Odor: OPAQUE THIN LIQUID WITH SOLVENT ODOR |  |  |  |  |  |  |  |  |
| SECTION 3 - FIRE AND EXPLOSION HAZARD DATA |  |  |  |  |  |  |  |  |
| FLAMMABILITY as per USA FLAME PROJECTION TEST NOT APPLICABLE | Auto Ignition Temperature N/E |  |  |  | Flammability Limits in Air by % in Volume: LOWER : 1.00 UPPER : 7.00 |  |  |  |
| FLASH POINT AND METHOD USED 145 F (TCC) SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus. |  | EXTINGUISHER MEDIA: Foam, dry chemical, carbon dioxide, water. |  |  |  |  |  |  |
| Unusual Fire & Explosion Hazards: TOXIC FUMES MAY BE EMITTED IF MATERIAL IS EXPOSED TO EXTREME TEMPERATURES. |  |  |  |  |  |  |  |  |
| SECTION 4 - REACTIVITY HAZARD DATA |  |  |  |  |  |  |  |  |
| STABILITY [ X ] STABLE [ ] UNSTABLE |  | HAZARDOUS POLYMERIZATION [ ] WILL [ X ] WILL NOT OCCUR |  |  |  |  |  |  |
| Incompatibility (Mat. to avoid): VERY STRONG OXIDIZING AGENTS |  | Conditions to Avoid: KEEP AWAY FROM HEAT OR OPEN FLAME. |  |  |  |  |  |  |
| Hazardous Decomposition Products: Smoke, Carbon Monoxide, Carbon Dioxide and other Toxic Fumes. |  |  |  |  |  |  |  |  |
| SECTION 5 - HEALTH HAZARD DATA |  |  |  |  |  |  |  |  |
| PRIMARY ROUTES OF ENTRY: [ X ] INHALATION [ X ] INGESTION [ X ] SKIN ABSORPTION [ X ] EYE [ ] NOT HAZARDOUS |  |  |  |  |  |  |  |  |
| ACUTE EFFECTS: None Known. |  |  |  |  |  |  |  |  |
| Inhalation: Prolonged exposure to vapors may cause signs & symptoms of Central Nervous System Depression such as Headache, Dizziness, Weakness and Loss of Coordination. Vapor/Mist may result in irritation to the entire Respiratory Tract. |  |  |  |  |  |  |  |  |
| Eye Contact: Contact with eyes may result in severe eye irritation and possible damage of the Corneal. Redness and Tearing. |  | Skin Contact: Prolonged or Repeated skin contact may result in skin irritation leading to Dermatitis. Defatting of skin may also occur. |  |  |  |  |  |  |
| Ingestion: Ingestion of this product may result in Gastrointestinal Irritation, Headache, Nausea, Vomiting and Diarrhea. |  |  |  |  |  |  |  |  |
| CHRONIC EFFECTS: None Known. |  |  |  |  |  |  |  |  |
| Medical Conditions Generally Aggravated by Exposure: None Known. |  |  |  |  |  |  |  |  |
| EMERGENCY FIRST AID PROCEDURES |  |  |  |  |  |  |  |  |
| Eye Contact: Remove contact lenses immediately. Flush eyes with plenty of water for at least 15 minutes. Get immediately medical attention. |  |  |  |  |  |  |  |  |
| Skin Contact: Wash skin with plenty of water. Remove contaminated clothing and wash before reuse. Get medical attention if needed. |  |  |  |  |  |  |  |  |
| Inhalation: Remove to Fresh Air. Give Oxygen or CPR. Get immediately medical attention. |  |  |  |  |  |  |  |  |
| Ingestion:. Do not give anything by mouth to an unconscious or convulsing person. Do not induce vomiting. Get immediately medical attention. |  |  |  |  |  |  |  |  |
| SECTION 6 - CONTROL AND PROTECTIVE MEASURES |  |  |  |  |  |  |  |  |
| Respiratory Protection (specify type): USE SELF CONTAINED AIR MASK IN CLOSED ROOMS. |  |  |  |  |  |  |  |  |
| Protective Gloves: Neoprene or Rubber Gloves. |  | Eye Protection: SAFETY GLASSES OR MONOGOGGLES. |  |  |  |  |  |  |
| Ventilation Requirements: GOOD VENTILATION SHOULD BE MAINTAINED. |  |  |  |  |  |  |  |  |
| Other Protective Clothing & Equipment: Impervious apron and boots. Provide emergency safety showers and eye wash fountain near work area. |  |  |  |  |  |  |  |  |
| Hygienic Work Practices: Do not mix this product with other chemicals. Toxic Fumes may be emitted. |  |  |  |  |  |  |  |  |
| SECTION 7 - PRECAUTIONS FOR SAFE HANDLING AND USE |  |  |  |  |  |  |  |  |
| Steps To Be Taken If Material Is Spilled Or Released: KEEP UNNECESSARY PEOPLE AWAY. SPILLS ARE SLIPPERY. DIKE & CONTAIN WITH INERT ABSORBENT MATERIAL. COLLECT FOR DISPOSAL. |  |  |  |  |  |  |  |  |
| Waste Disposal Methods: CONSULT LOCAL AUTHORITY FOR ALTERNATE METHODS. OBSERVE ALL LOCAL, STATE & FEDERAL REGULATIONS. |  |  |  |  |  |  |  |  |
| Precautions To Be Taken In Handling & Storage: STORE IN A COOL WELL VENTILATED AREA AWAY FROM SPARK, HEAT AND OPEN FLAME. |  |  |  |  |  |  |  |  |
| Other Precautions &/or Special Hazards: KEEP OUT OF REACH OF CHILDREN. |  |  |  |  |  |  |  |  |

We believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind.

** Chemical Listed as Carcinogen or Potential Carcinogen. [a] NTP [b] IARC Monograph [c] OSHA [d] Not Listed [e] Animal Data Only