MATERIAL SAFETY DATA SHEET

This MSDS complies with OSHA'S Hazard Communication Standard 29 CFR 1910.1200 and OSHA Form 174

| IDENTITY AND MANUFACTURER'S INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NFPA Rating: Health-1 ; Flammability- 0; Reactivity-0 ; Special-0 |  | HMIS Rating: Health- 1 Flammability-0 ; Reactivity- 0; Personal Protection- 0 |  |  |  |  |  |  |
| MANUFACTURED BY : MISSION LABORATORIES Address: 2433 BIRKDALE ST LOS ANGELES, CA 90031 |  | D OT Hazard Classification: NON HAZARDOUS Identity (trade name as used o n label): RESTORE 50:1 DRI-FOAM SHAMPOO |  |  |  |  |  |  |
| Date Prepared 06/23/04 Prepared By: MM |  | MSDS Number: F1960 Revision- |  |  |  |  |  |  |
| Information Calls: (323) 223-1405 EMERGENCY RESPONSE NUMBER: (800) 255-3924 |  | NOTICE: JUDGMENT BASED ON INDIRECT T EST DATA |  |  |  |  |  |  |
| SECTION 1 - MATERIAL IDENTIFICATION AND INFORMATION |  |  |  |  |  |  |  |  |
| COMPONENTS-CHEMICAL NAMES AND COMMON NAMES (Hazardous Components 1% or greater; Carcinogens 0.1% or greater) |  |  | CAS Number | SARA III LIST |  | OSHA PEL (ppm) | ACGIH TLV (ppm) | Carcinogen Ref. Source ** |
| 2-BUTOXYETHANOL 5.00 |  |  | 111-76-2 | N/E |  | 25 PPM | 25 PPM | N/E |
| SECTION 2 - PHYSICAL/CHEMICAL CHARACTERISTICS |  |  |  |  |  |  |  |  |
| Boiling Point: >150 F |  | Specific Gravity: 1.048 |  |  |  |  |  |  |
| Vapor Pressure: N/E Max. |  | pH of product: 9.00 |  |  |  |  |  |  |
| Vapor Density N/E |  | Evaporation Rate : <1.00 |  |  |  |  |  |  |
| Solubility in Water: Soluble |  | V.O.C. : 57 g / L |  |  |  |  |  |  |
| Appearance and Odor: CLEAR THIN LIQUID W/ A PLEASANT ODOR |  |  |  |  |  |  |  |  |
| SECTION 3 - FIRE AND EXPLOSION HAZARD DATA |  |  |  |  |  |  |  |  |
| FLAMMABILITY as per USA FLAME PROJECTION TEST NON FLAMMABLE | Auto Ignition Temperature |  |  |  | Flammability Limits in Air by % in Volume: LOWER: 1.1 - UPPER: 12.7 |  |  |  |
| FLASH POINT AND METHOD USED (non-aerosols): > 200 F (TCC) SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus. |  | EXTINGUISHER MEDIA: Foam, dry chemical, carbon dioxide, water. |  |  |  |  |  |  |
| Unusual Fire & Explosion Hazards: Container may explode in Heat of Fire. |  |  |  |  |  |  |  |  |
| SECTION 4 - REACTIVITY HAZARD DATA |  |  |  |  |  |  |  |  |
| STABILITY [ X ] STABLE [ ] UNSTABLE |  | HAZARDOUS POLYMERIZATION [ ] WILL [ X ] WILL NOT OCCUR |  |  |  |  |  |  |
| Incompatibility (Mat. to avoid): VERY STRONG OXIDIZING AGENTS |  | Conditions to Avoid: Heat, Sparks, Flame and Ignition. |  |  |  |  |  |  |
| Hazardous Decomposition Products: Burning may produce Carbon Monoxide, Carbon Dioxide and Hydrocarbons. |  |  |  |  |  |  |  |  |
| SECTION 5 - HEALTH HAZARD DATA |  |  |  |  |  |  |  |  |
| PRIMARY ROUTES OF ENTRY: [ X ] INHALATION [ X ] INGESTION [ X ] SKIN ABSORPTION [ X ] EYE [ ] NOT HAZARDOUS |  |  |  |  |  |  |  |  |
| ACUTE EFFECTS : None known. |  |  |  |  |  |  |  |  |
| Inhalation: Excessive inhalation at high concentration are irritating to the nose, eyes, and respiratory tract. May cause headache, dizziness, nausea, vomiting and malaise. |  |  |  |  |  |  |  |  |
| Eye Contact: Eye contact may cause severe irritation with possible corneal injury. |  | Skin Contact: Prolonged widespread contact with saturated clothing may cause reddening, swelling, drying and possible damage to skin. |  |  |  |  |  |  |
| Ingestion: Moderately Toxic. May cause headache, dizziness, nausea, vomiting, diarrhea and general weakness. Ingestion may cause Red Blood Cell Hemolysis and possible Kidney and Liver Injury. |  |  |  |  |  |  |  |  |
| CHRONIC EFFECTS: None known.. |  |  |  |  |  |  |  |  |
| Medical Conditions Generally Aggravated by Exposure: None Known. |  |  |  |  |  |  |  |  |
| EMERGENCY FIRST AID PROCEDURES |  |  |  |  |  |  |  |  |
| Eye Contact: Contact lenses should be removed immediately flush eyes for at least 15 minutes with plenty of water. Seek immediately medical attention. |  |  |  |  |  |  |  |  |
| Skin Contact: Flush skin thoroughly with plenty of water. Change contaminated clothing and launder before reuse. Seek medical attention if irritation persist. |  |  |  |  |  |  |  |  |
| Inhalation: Remove to fresh air immediately Give Oxygen or Artificial Respiration if needed. Seek medical attention immediately. |  |  |  |  |  |  |  |  |
| Ingestion: Do not give anything by mouth to an unconscious or convulsing person. Induce vomiting. Give 2 glasses of water to drink and get immediately medical attention. |  |  |  |  |  |  |  |  |
| SECTION 6 - CONTROL AND PROTECTIVE MEASURES |  |  |  |  |  |  |  |  |
| Respiratory Protection (specify type): self-contained breathing apparatus. Or Vapor Respirator. |  |  |  |  |  |  |  |  |
| Protective Gloves: Rubber of Neoprene Gloves. |  | Eye Protection: SAFETY GLASSES |  |  |  |  |  |  |
| Ventilation Requirements: GOOD VENTILATION SHOULD BE MAINTAINED. Mechanical/Local Exhaust may be needed. |  |  |  |  |  |  |  |  |
| Other Protective Clothing & Equipment: Impervious apron/clothing and boots. Provide emergency safety shower and eye fountain near work area. |  |  |  |  |  |  |  |  |
| Hygienic Work Practices: Do not mix with other chemicals. |  |  |  |  |  |  |  |  |
| SECTION 7 - PRECAUTIONS FOR SAFE HANDLING AND USE |  |  |  |  |  |  |  |  |
| Steps To Be Taken If Material Is Spilled Or Released: KEEP UNNECESSARY PEOPLE AWAY. SPILLS ARE SLIPPERY. DIKE & CONTAIN WITH INERT ABSORBENT MATERIAL. COLLECT FOR DISPOSAL. Wear suitable protective clothing and equipment. |  |  |  |  |  |  |  |  |
| Waste Disposal Methods: CONSULT LOCAL AUTHORITY FOR ALTERNATE METHODS. OBSERVE ALL LOCAL, STATE & FEDERAL REGULATIONS. |  |  |  |  |  |  |  |  |
| Precautions To Be Taken In Handling & Storage: STORE IN A DRY COOL AREA AWAY FROM freezing, heating AND OPEN FLAME.. |  |  |  |  |  |  |  |  |
| Other Precautions &/or Special Hazards: KEEP OUT OF REACH OF CHILDREN. |  |  |  |  |  |  |  |  |

We believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind.

**  Chemical Listed as Carcinogen or Potential Carcinogen. [a] NTP [b] IARC Monograph [c] OSHA [d] Not Listed [e] Animal Data Only