|  | Spartan Chemical Company, Inc. Material Safety Data Sheet |
| --- | --- |

SECTION I: PRODUCT INFORMATION SECTION I: PRODUCT INFORMATION

| Product Name or Number (as it appears on label): | Product Division: |
| --- | --- |
| CLEAN ON THE GO DAMP MOP [8] Product Number: 4736 | Janitorial |

| Spartan Chemical Company, Inc. | Product/Technical Information: | 1-(800)-537-8990 |
| --- | --- | --- |
| 1110 Spartan Drive | Medical Emergency: | 1-(888)-314-6171 (24 hours) |
| Maumee OH 43537 | Chemical Leak/Spil Emergency: | CHEMTREC 1-(800) 424-9300 (24 hours) |

| Shipping Description: | Cleaning compounds, liquid, n.o.s. |
| --- | --- |

| NFPA Ratings: |  | HMIS Ratings: |  |
| --- | --- | --- | --- |
| Health: | 1- Slight | Health: | 1- Slight |
| Fire: | 0 - Minimal | Fire: | 0 - Minimal |
| Reactivity: | 0 - Minimal | Reactivity: | 0 - Minimal |
|  |  | Pers. Prot. Equip.: | See Section VIII |

SECTION II: HAZARDOUS INGREDIENTS 
SECTION II: HAZARDOUS INGREDIENTS
(Listed when present at 1% or greater, carcinogens at 0.1% or greater) All component chemicals are listed or exempted from listing on the "TSCA Inventory" of chemical substances maintained by the U.S. Environmental Protection Agency.

| Chemical Name(s) | % W | CAS Registry No. | Table Z- 1 - A |  |  | NTP, IARC or OSHA Carcinogen |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TWA mg/m3 | STEL mg/m 3 | CELING mg/m3 |  |
| Nonylphenol ethoxylate | 1-5 | 9016-45-9 | Not Established | Not Established | Not Established | No |
| Isopropyl alcohol | 1-5 | 67-63-0 | 980 | 1225 | Not Established | No |

SECTION III: PHYSICAL DATA 
SECTION III: PHYSICAL DATA

| Boiling Point: | 212 °F | Vapor Pressure: | Not determined |
| --- | --- | --- | --- |
| Vapor Density (AIR = 1): | Unknown | Solubility in Water: | Complete |
| pH: | 7.0-8.0 | Specific Gravity (H2O=1): | 1.00 |
| Evaporation Rate (but.ace.=1): | <1 | Percent Solid by Weight: | 4-6 |
| Physical State: | Liquid |  |  |

| Appearance & Odor: | Yellow liquid, lemon scent |
| --- | --- |

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA 
SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

| Flash Point: | > 212°F | Method Used: | ASTM-D56 |
| --- | --- | --- | --- |
| Flammable Limits: | Not Established | Flame Extension: | N/A |

| Extinguishing Media: | Product does not support combustion. Use extinguishing media appropriate for surrounding fire. |
| --- | --- |
| Special Fire Fighting Procedures: | Wear NIOSH approved self-contained breathing apparatus and protective clothing. Cool fire-exposed containers with water spray. |
| Unusual Fire & Explosive Hazards: | Combustion products are toxic. |

SECTION V: HEALTH HAZARD DATA 
SECTION V: HEALTH HAZARD DATA

| Threshold Limit Value: | Not Established | Primary Routes of Entry: | Inhalation, Skin Contact, Eyes; Oral |
| --- | --- | --- | --- |

| Effects of Overexposure- Conditions to Avoid: | Causes mild eye irritation: Symptoms may include pain, redness, swelling and tearing May cause skin irritation: Symptoms may include pain and redness. May be harmful if swallowed: Symptoms may include nausea, vomiting, pain and diarrhea. Inhalation of product mist may cause respiratory irritation: Symptoms may include coughing and difficulty breathing. Avoid contact with eyes, skin or clothing. Do not swallow. Avoid breathing product mist. Wash thoroughly after handling. |
| --- | --- |
| Conditions Aggravated by Use: | Use of this product may aggravate preexisting skin; eye and respiratory disorders including asthma and dermatitis. |

Emergency & First Aid Procedures:

| Eyes: | Immediately flush eyes with plenty of water for at least 15 minutes. Remove contact lenses. Get medical attention. |
| --- | --- |
| Skin: | Remove contaminated clothing. Wash skin thoroughly with soap and water. Get medical attention if irritation persists. Wash contaminated clothing before reuse. |
| Ingestion: | Do not induce vomiting unless told to do so by medical personnel. Drink one or two glasses of water to dilute product. Get medical attention. Do not give anything by mouth to an unconscious person. |
| Inhalation: | In case of respiratory irritation; move person to fresh air. Get medical attention if irritation persists. |

SECTION VI: REACTIVITY DATA 
SECTION VI: REACTIVITY DATA

| Stability: | Stable | Incompatible Materials: | Strong oxidizers |
| --- | --- | --- | --- |
| Hazardous Decomposition Products: | None known | Hazardous Polymerization: | Will Not Occur |

SECTION VII: SPILL OR LEAK PROCEDURES

SECTION VII: SPILL OR LEAK PROCEDURES

| Steps to be Taken in Case Material is Released or Spilled: | Small spills of one gallon or less may be flushed with plenty of water to sanitary sewer system (If permitted by local sewer regulations). Dike and contain large spills with inert material and transfer liquid to containers for recovery or disposal. |
| --- | --- |
| Waste Disposal Method: | Assure conformance with federal, state and local regulations. |

SECTION VIII: SPECIAL PROTECTION INFORMATION 
SECTION VIII: SPECIAL PROTECTION INFORMATION

| Respiratory Protection: | Not normally required. However if respiratory irritation occurs; the use of a NIOSH approved respirator suitable for the use-conditions and chemicals in Section II should be considered. |
| --- | --- |
| Ventilation: | Provide good general ventilation. Local exhaust ventilation may be necessary for some operations. |
| Protective Gloves(Specify Type): | Impervious rubber or other waterproof gloves are recommended for frequent or prolonged contact with undiluted product. |
| Eye Protection(Specify Type): | Splash goggles are recommended when handling undiluted product. |
| Other Protective Equipment: | See 29 CFR 1910.132-138 for further guidance. |

SECTION IX: SPECIAL PRECAUTIONS 
SECTION IX: SPECIAL PRECAUTIONS

| Precautions; Handling & Storing: | Keep out of reach of children. |
| --- | --- |
| Other Precautions: | California Proposition 65 : WARNING: This product contains trace amounts of a chemical known to the State of California to cause cancer or birth defects or other reproductive harm. |

| © SCC 11/15/2003 | Name: | Ronald T. Cook | Title: | Manager, Regulatory Affairs |
| --- | --- | --- | --- | --- |
| CLEAN ON THE GO DAMP MOP [8] | Effective Date: | 11/15/2003 | Supercedes: | 11/11/1999 |
| Ref: 29 CFR 1910.1200 (OSHA) | Changes: | Update Sections I, II, III, IV, V, VII, VIII & IX |  |  |

This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained herein. Actual conditions of use and handling are beyond sellers control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State and Local laws and regulations.