# SAFETY DATA SHEET

DATE: November 3, 2023

CONTINENTAL CHEMICAL CO.	EMERGENCY PHONE

4920 E. WASHINGTON BLVD.		TRANSPORTATION:CHEMTREC: 800-424-9300

COMMERCE CA. 90040 			NON-TRANSPORTATION: HJB INC: 323-268-1288

---------------------------------------------------------------------------------------------------------------------------------

SECTION I GENERAL INFORMATION

PRODUCT NAME:    CIRCULATION

MANUFACTURER:	CONTINENTAL CHEMICAL

HMIS: 301 X

# SECTION II- HAZARDS IDENTIFICATION

CLASSIFICATION

GHS CLASSIFICATION:			SKIN CORROSION (CATEGORY 1A)

SERIOUS EYE DAMAGE (CATEGORY 1)

ACUTE AQUATIC TOXICITY (CATEGORY 3)

GHS LABEL ELEMENTS

PICTOGRAM:

SIGNAL WORD:			DANGER

HAZARD STATEMENT:	H314: CAUSES SEVERE SKIN BURNS AND EYE DAMAGE

H402: HARMFUL TO AQUATIC LIFE

PRECAUTIONARY STATEMENTS:	P280:WEAR PROTECTIVE GLOVES/PROTECTIVE CLOTHING/EYE PROTECTION/FACE PROTECTION.

P303/P361/P353: IF ON SKIN OR HAIR: REMOVE ALL CONTAMINATED CLOTHING. RINSE SKIN WITH WATER/SHOWER

P305/P351/P338: IF IN EYES: RINSE CAUTIOUSLY WITH WATER FOR SEVERAL MINUTES. REMOVE CONTACT LENSES. CONTINUE RINSING.

P313: GET MEDICAL ATTENTION.

HMIS CLASSIFICATION:	HEALTH HAZARD:	3

FLAMMABILITY:	0

PHYSICAL HAZARDS:	1

NFPA RATING:	HEALTH HAZARD:	3

FIRE:			0

REACTIVITY HAZARD:	0

POTENTIAL HEALTH EFFECTS

INHALATION:	MAY BE HARMFUL IF INHALED. MATERIAL IS EXTREMELY DESTRUCTIVE TO THE TISSUE OF THE MUCOUS MEMBRANE AND UPPER RESPIRATORY TRACT.

SKIN:	MAY BE HARMFUL IF ABSORBED THROUGH SKIN. CAUSE SEVERE SKIN BURNS.

EYES:	CAUSE SEVERE EYE BURNS.

INGESTION:	TOXIC IF SWALLOWED. DO NOT INGEST.

SECTION III CHEMICAL COMPOSITION

COMPONENT						CAS#		CONCENTRATION

SODIUM HYDROXIDE					1310-73-2		35%

# SECTION IV FIRST AID MEASURES

IF INHALED:                                                 IF INHALED, MOVE PERSON INTO FRESH AIR. IF NOT

BREATHING, GIVE ARTIFICIAL RESPIRATION.

CONSULT A PHYSICIAN.

SKIN CONTACT:                                          TAKE OFF CONTAMINATED CLOTHING AND SHOES

IMMEDIATELY. FLUSH SKIN WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES. CONSULT A PHYSICIAN.

EYE CONTACT:	FLUSH EYES WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES AND CONSULT A PHYSICIAN.

IF INGESTED:	DO NOTINDUCE VOMITING UNLESS INSTRUCTED TO DO SO BY A PHYSICIAN. NEVER GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON. RINSE MOUTH EITH WATER. CONSULT A PHYSICIAN.

SECTION V FIREFIGHTING MEASURES

FLAMMABILITY OVERVIEW:	CONSIDERED A NON-FLAMMABLE WATER SOLUTION.

FLASH POINT:	N/A

EXTINGUISHING MEDIA:	USE WATER-SPRAY, ALCOHOL-RESISTANT FOAM, DRY CHEMICAL, OR CARBON DIOXIDE.

SPECIAL PROTECTIVE EQUIPMENT:	WEAR A SELF-CONTAINED BREATHING APPARATUS (SCBA) FOR FIREFIGHTING, IF NECESSARY.

HAZARDOUS COMBUSTION PRODUCTS: HAZARDOUS DECOMPOSITION PRODUCTS UNDER FIRE CONDITION: SODIUM OXIDES.

SECTION VI ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS:	USE PERSONAL PROTECTIVE EQUIPMENT. AVOID BREATHING VAPORS, MIST OR GAS. ALWAYS ENSURE ADEQUATE VENTILATION. EVACUATE PERSONNEL TO SAFE AREAS.

ENVIRONMENTAL PRECAUTIONS:	PREVENT FURTHER LEAKAGE OR SPILLIAGE. PREVENT FROM ENTERING DRAINS AND WATERWAYS. DISCHARGE INTO THE ENVIRONMENT MUST BE AVOIDED.

CONTAINMENT AND CLEAN UP:	SOAK UP WITH INERT ABSORBENT MATERIAL AND DISPOSE OF AS HAZARDOUS WASTE. KEEP IN SUITABLE, CLOSED CONTAINERS FOR DISPOSAL.

# SECTION VII HANDLING AND STORAGE

SAFE HANDLING:	USE PERSONAL PROTECTIVE EQUIPMENT. AVOID BREATHING VAPORS, MIST OR GAS. ALWAYS ENSURE ADEQUATE VENTILATION.

SAFE STORAGE:	KEEP CONTAINERS TIGHTLY CLOSE IN A DRY WELL VENTILATED PLACE. CONTAINERS WHICH ARE OPENED MUST BE CAREFULLY RESEALED AND KEPT UPRIGHT TO PREVENT LEAKAGE. STORE AWAY FROM INCOMPATIBLE MATERIALS.

# SECTION VIII EXPOSURE CONTROLS/ PERSONAL PROTECTION

EXPOSURE LIMITS

COMPONENT			CAS NUMBER	EXPOSURE LIMIT		BASIS

SODIUM HYDROXIDE		1310-73-2		2 mg/m3			ACGIH-TLV

2 mg/m3			OSHA TABLE Z-1

2 mg/m3			NIOSH EXPOSURE LIMITS

PERSONAL PROTECTIVE EQUIPMENT

EYE PROTECTION:	WEAR TIGHTLY FITTING SAFETY GOGGLES AND FACE SHIELD. USE EQUIPMENT FOR EYE PROTECTION TESTED AND APPROVED UNDER NIOSH. HAVE EYE-WASH STATIONS AVAILABLE WHERE EYE CONTACT CAN OCCUR.

HAND PROTECTION:	HANDLE WITH CHEMICAL-RESISTANT GLOVES. GLOVES MUST BE INSPECTED PRIOR TO USE. DISPOSE OF CONTAMINATED GLOVES. WASH AND DRY HANDS AFTER USE.

SKIN PROTECTION:	WEAR COMPLETE SUIT PROTECTION AGAINST CHEMICALS, INCLUDING CHEMICAL-RESISTANT BOOTS. SAFETY SHOWERS SHOULD BE LOCATED IN THE WORK AREA WHERE SKIN CONTACT CAN OCCUR.

RESPIRATORY PROTECTION:	WHERE RISK ASSESSMENT SHOWS AIR-PURIFYING RESPIRATORS ARE APPROPRIATE, USE A NIOSH-APPROVED FULL FACE RESPIRATOR WITH APPROPRIATE CARTRIDGES. RESPIRATORY PROTECTION MUST BE PROVIDED IN ACCORDANCE WITH OSHA 29 CFR 1910.134.

GENERAL CONTROLS:	ALWAYS ENSURE ADEQUATE VENTILATION AND THAT WORKING AREAS CONTAINS SAFETY SHOWERS AND EYE WASH STATIONS. HANDLE MATERIAL IN ACCORDANCE WITH GOOD INDUSTRIAL HYGIENE AND SAFETY PRACTICES.

SECTION IX PHYSICAL AND CHEMICAL PROPERTIES

PHYSICAL STATE:	LIQUID

COLOR:	COLORLESS

ODOR:	TYPICAL

pH (as is):	12.5 – 13.5

FREEZING POINT:	<10°C

BOILING POINT:	>130°C

VAPOR DENSITY (air = 1):	> 1

SPECIFIC GRAVITY:	1.37 @ 25°C

SOLUBILITY IN WATER:	COMPLETE

SECTION X STABILITY AND REACTIVITY

CHEMICAL STABILITY:	STABLE UNDER RECOMMENDED STORAGE CONDITIONS.

MATERIALS TO AVOID:	AVOID WATER, ACIDS, STRONG OXIDIZING AGENTS, MOST METALS (ALUMINUM, LEAD, TIN, ZINC).

HAZARDOUS DECOMPOSITION:	SODIUM OXIDES

SECTION XI TOXICOLOGICAL INFORMATION

ACUTE TOXICITY

COMPONENT			CAS NUMBER	TEST			TOXICITY

SODIUM HYDROXIDE		1310-73-2		ORAL LD50		NO DATA AVAILABLE

DERMAL LD50		NO DATA AVAILABLE

INHALATION LC50 		NO DATA AVAILABLE

POTENTIAL HEALTH EFFECTS

INHALATION:	MAY BE HARMFUL IF INHALED. MATERIAL IS EXTREMELY DESTRUCTIVE TO THE TISSUE OF THE MUCOUS MEMBRANE AND UPPER RESPIRATORY TRACT.

SKIN:	MAY BE HARMFUL IF ABSORBED THROUGH SKIN. CAUSES SKIN BURNS.

EYES:	CAUSES SEVERE EYE BURNS.

INGESTION:	TOXIC IF SWALLOWED. DO NOT INGEST.

SYMPTOMS OF EXPOSURE:	BURNING SENSATION, NAUSEA, COUGH, WHEEZING, LARYNGITIS, SHORTNESS OF BREATH, SPASM, INFLAMMATION, AND EDEMA OF THE LARYNX. SPASM, INFLAMMATION, AND EDEMA OF THE BRONCHI, PNEUMONITIS, PULMONARY EDEMA, HEADACHE.

CHRONIC EFFECTS OF EXPOSURE:	NO DATA AVAILABLE

CARCINOGENICITY:	NO COMPONENTS OF THIS PRODUCT AT LEVELS GREATER THAN 0.1% IS IDENTIFIED AS CARCINOGENIC BY IARC, ACGIH, OR OSHA.

SECTION XII ECOLOGICAL INFORMATION

ACUTE ECOTOXICITY

COMPONENT			CAS NUMBER	ORGANISM		ECOTOXICITY

SODIUM HYDROXIDE		1310-73-2		BLUEGILL SUNFISH	48 hr LC50 =99mg/L

ECOLOGICAL EFFECTS

PERSISTENCE AND DEGRADABILITY:	NO DATA AVAILABLE

BIOACCUMULATION POTENTIAL:	NOT EXPECTED TO BIO-ACCUMULATE

MOBILITY IN SOIL:			NO DATA AVAILABLE

OTHER ADVERSE EFFECTS:	HARMFUL TO AQUATIC LIFE DUE TO THE INCREASE pH OF THE WATER.

SECTION XIII DISPOSAL

DISPOSAL:	SURPLUS AND NON-RECYCLABLE MATERIAL SHOULD BE TREATED AS HAZARDOUS WASTE AND BE DISPOSED OF BY A LICENSED DISPOSAL COMPANY. DO NOT RELEASE INTO SEWERS OR WATERWAYS.

CONTAMINATED PACKAGING:	DISPOSE OF AS UNUSED PRODUCT.

SECTION XIV TRANSPORTATION

PRODUCT NAME:			CIRCULATION

PROPER SHIPPING NAME:		SODIUM HYDROXIDE SOLUTION

ID NUMBER:				UN 1824

HAZARD CLASS OR DIVISION:	8

PACKING GROUP:			II

DOT HAZARDOUS SUBSTANCE:	SODIUM HYDROXIDE 1000 lbs (454 Kgs)

SECTION XV REGULATORY INFORMATION

US FEDERAL

SARA 302 COMPONENTS:		NONE LISTED

SARA 311/312 HAZARDS:		ACUTE HEALTH HAZARD

SARA 313 COMPONENTS:		NONE LISTED

TSCA INVENTORY:			SODIUM HYDROXIDE (CAS 1310-73-2)

EUROPEAN UNION

EC INVENTORY:			SODIUM HYDROXIDE (EINECS 215-185-5)

STATE REGULATIONS

CA PROP 65:				NONE LISTED

SECTION XVI OTHER INFORMATION

SDS VERSION:				1.0

REVISION DATE:			05/12/2015

SUPERSEDES:				NEW TO GHS

THE MANUFACTURER BASED ON ITS OWN DATA AND THE WORK OF OTHERS HAS ASSEMBLED THE INFORMATION IN THIS DATA SHEET. THE MANUFACTUER MAKES NO WARRANTIES, EXPRESS OR IMPLIED, AS TO THE ACCURACY, COMPLETENESS, OR ADEQUACY OF THE INFORMATION CONTAINED HEREIN. THE MANUFACTURER SHALL NOT BE LIABLE (REGARDLESS OF FAULT) TO VENDEE, THE VENDEE’S EMPLOYEES, OR ANYONE FOR ANY DIRECT, SPECIAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE ACCURACY, COMPLETENESS, ADEQUACY, OR FURNISHING OF SUCH INFORMATION.