MATERIAL SAFETY DATA SHEET

Product Name: Millenium Super Bowl Brite High Acid Bowl Cleaner	Formula Number:  303-001.95

Section One		Product and Company Information

Product Name:  Millenium Super Bowl Brite High Acid Bowl Cleaner

Chemical Family:  Acid			Application:  Acid toilet bowl cleaner	CAS#:  Mixture

Manufacturer:		Canberra Corporation		Issue Date:  June 1, 1997

3610 Holland-Sylvania Rd. 	Previous Revision Date:  June 1, 1995

Toledo, Ohio 43615 		Transportation Emergency:  Chemtrec 1-800-424-9300

Product Technical Information:  1-419-841-6616	Medical/Handling Emergency:  1-419-841-6616

Section Two		Composition/Information on Ingredients

Component				Hazard		CAS#		OSHA PEL	ACGIH TLV	%

Hydrochloric Acid				Corrosive		7647-01-0		5 ppm(C)		5 ppm(C)		23.0

Water					N/A		7732-18-5		N/A		N/A		75.0

Nonoxynol				Eye irritant	9016-45-9		N/A		N/A		1.0

Section Three		Hazards Identification

Statement of Hazards:  Danger:  Causes eye and skin damage.  Corrosive Liquid.  Overexposure may cause severe irritation and burning of affected areas.

Fire and Explosion Hazards:  This product is water based and represents little or no fire hazard at room temperature.  Thermal decomposition products may include carbon monoxide, carbon dioxide, nitrogen oxides and hydrogen chloride or chlorine gas.

Primary Route of Exposure:  Skin and eye contact are the principal routes of exposure to this product.

Skin Contact - Skin contact can cause severe irritation or burns with possible redness, swelling or blistering. Severity of action is highly dependent on contact time.

Eye Contact - Eye contact may cause severe irritation or burns.  May cause permanent eye damage, if not promptly flushed with plenty of  water.

Ingestion - If swallowed, this product may cause severe irritation or damage of mucosal linings of the mouth, throat, esophagus or stomach.

Inhalation - Inhalation can cause irritation and burning of respiratory passages.

Carcinogenicity:	IARC - No		NTP - No		OSHA - No	ACGIH - No

Section Four		First Aid Measures

Skin Contact:  Immediately remove contaminated shoes and clothing.  Wash skin with soap and plenty of water for 15 minutes.  Do not attempt to neutralize with chemical agents.  Get medical attention. Wash contaminated clothing before reuse.

Eye Contact:  Immediate first aid is required to prevent eye damage.  Flush immediately with water and remove contact lenses, if applicable.  Then flush with large quantities of  running water for 15 minutes.  Hold eyelids apart during flushing to ensure complete rinsing of entire surface of eye and lids with water.  Do not attempt to neutralize with chemical agents. Get medical attention immediately.  Continue flushing with water if physician is not immediately available.

Ingestion:  DO NOT induce vomiting.  Rinse mouth with water.  Then give 1-2 glasses of water or milk and call a physician or poison control center immediately.  If vomiting occurs, keep head below hips to reduce risk of aspiration.  Give water again.  NEVER give anything by mouth to a person who is unconscious or convulsing.  Get medical attention immediately.

Inhalation:  Remove victim to fresh air.  If not breathing, give artificial respiration. If breathing is difficult, give oxygen.  Get medical attention.

Medical Conditions Aggravated:  No data is available that addresses medical conditions that are generally recognized as being aggravated by exposure to this product.	Note to Physician:  Treat exposed patients symptomatically.

Section Five		Fire Fighting Measures

Flash Point(Method):  None							Flammable Limits:  Not applicable.

Extinguishing Media:  Use water spray, dry chemical, carbon dioxide or foam extinguishing agents unless contraindicated by surrounding environment.

Special Fire Fighting Procedures:  Prevent human exposure to fire, smoke, fumes or products of combustion.  Evacuate non-essential personnel from fire area.  All ignition sources should be extinguished.  Fire fighters should wear appropriate protective equipment, including self contained breathing apparatus and impervious clothing.  Keep containers cooled with water spray to prevent rupture from excess heat.

Fire and Explosion Hazards:  No unusual fire or explosion hazards of this product are known.

Hazardous Products:  Thermal decomposition of this product may generate oxides of carbon, nitrogen and hydrogen chloride.

Section Six		Accidental Release Measures

Caution:  Ventilate area immediately.  If spilled, floors may be extremely slippery.  Wear appropriate protective gear to avoid skin or eye contact.  Dike and contain spill with inert material(sand, clay).  Transfer liquid and solids to separate containers for recovery or proper disposal.  Keep spill out of storm sewers and open bodies of water. After removal, flush area with water.  Follow good industrial hygiene practices.  Wash thoroughly after clean-up.  If uncertain about proper disposal, contact your local waste disposal provider or the regional office of the Environmental Protection Agency for guidance.

Section Seven		Handling and Storage

Handling:  Avoid contact with skin or eyes.  Avoid breathing vapor, spray or mist.  Use product only according to label directions.  If unsure about safe use, contact your supervisor immediately.  Wash with soap and water after handling.

Storage:  Store in original container in a cool, dry, well ventilated area away from sources of ignition.  Store away from food or feed.  Containers should be kept tightly closed when not in use.  Keep in an area inaccessible to small children.

Section Eight		Exposure Controls/Personal Protection

Personal Protective Equipment

Respiratory:  Respiratory protection is not expected to be necessary under normal conditions of use.  Where exposure cannot be adequately controlled by general or local ventilation, use appropriate respiratory protection to prevent overexposure.  An approved acid gas respirator would be recommended.

Gloves:  Use water impervious gloves, such as latex or neoprene rubber.

Eye Protection:  Chemical resistant goggles or face shield are recommended.

Other:  Protective clothing(long sleeves, pants), eye wash, safety shower are always advisable when working with chemicals.

Engineering Controls

Ventilation:  In applications where sprays or mists may be generated, proper ventilation in accordance with good industrial hygiene should be provided.

Section Nine		Physical/Chemical Properties

Form			Liquid						pH			< 1

Color			Pink						Specific Gravity	1.118

Odor			Pungent						Solubility		Completely soluble

Boiling Point		190-212ºF					Freezing Point		Not Established

Evaporation Rate	> water						% Volatile		> 95%

Section Ten		Stability and Reactivity

Stability:  This product is stable at ambient temperatures and atmospheric pressures.  It is not self-reactive and has a shelf life of at least one year under sealed conditions.  Conditions to avoid are temperatures above 130º F or below 32º F.  Do not mix with other chemicals.  Hazardous conditions may arise from improper mixing  of chemicals.  Mixing with bleach or other chlorine sources may generate potentially toxic gases.

Hazardous Decomposition Products:  Thermal decomposition may generate toxic materials such as oxides of carbon,nitrogen, or hydrogen chloride.

Dangerous Polymerization:  Will not occur.  Conditions to avoid: None

Dangerous Reactions:  May react with strong bases to give exothermic conditions.  Reacts with bleach or other chlorine sources to generate toxic chlorine gas.

Section Eleven		Toxicology Information

Acute Toxicity:  This product has not been evaluated for its acute toxicology profile.

Chronic Toxicity:  This product has not been evaluated for its chronic toxicology profile.

Carcinogenicity:  This product does not contain a listed carcinogen from NTP, ACGIH or IARC.

Irritation:  This product can be expected to be a skin and eye corrosive.  No skin sensitization data is currently available.

Section Twelve		Ecology Information

Ecotoxicity:  The ecological toxicity of this product is not known.

Section Thirteen		Disposal Considerations

Waste Disposal:  Material that cannot be used according to label direction and is destined for disposal should be disposed of in accordance with all applicable regulations.  Empty containers should be triple rinsed and offered for recycling or reconditioning, or disposed of in a sanitary landfill or for small containers(1 gallon or less), may be wrapped and discarded in trash.  Waste generators are required to evaluate all waste material to determine if it is hazardous by characteristics or listing for compliance with RCRA and any local disposal procedures and regulations.

NOTE:  State and local regulations may be more stringent than Federal regulations.

Section Fourteen		Transport Information

DOT 		Proper Shipping Name:	Consumer Commodity			Hazard Class:		ORM-D

Packing Group:		II					ID Number:		UN1789

DOT Label:		ORM-D					Emergency Guide No.:	60

Emergency Phone No.:	CHEMTREC  1-800-424-9300

Section Fifteen		Regulatory Information

FIFRA:  This is a registered product, EPA Reg. No. 8155-3.

Toxic Substances Control Act(TSCA):  All of the components intentionally added to this product are listed on the on the U.S. EPA TSCA inventory.

SARA Title III

Section 302:  This product contains the following chemicals subject to SARA 302 reporting: None.

Section 311/312:  This product is classified as an "immediate (acute) health hazard".

Section 313:  *Chemicals marked with an asterisk in Section Two are subject to the reporting requirements for Section 313 of Title III of the Superfund Amendments and Reauthorization Act(SARA) of 1986 and 40CFR Part 372.  However, if this product is utilized for routine janitorial maintenance, it is exempted  from this section, as stated in paragraph B.3.b Activity Exemptions of the Form R instructions.

OSHA Hazard Communication Standard(29 CFR 1910.1200):  This product is hazardous by definition of the Hazard Communication Standard.

Canada Domestic Substance List:  All of the ingredients intentionally added to this product are listed on the DSL.

Canada WHMIS:  Class E

NPCA  Hazardous Material Identification System (HMIS Rating)

Health  3		Flammability  0		Reactivity  1		Personal Protective Eqpt.  H

State Regulations

Pennsylvania/New Jersey/Massachusetts Right to Know

See "Section Two Composition" for listing of  hazardous and top five ingredients present in concentration greater than 1%.

California Proposition 65:  Components of this product which are present at a concentration which could require reporting under this statute are:  None.

Section Sixteen		Other Information

Disclaimer:  No representations or warranties, either expressed or implied, of merchantability, fitness for a particular purpose, or of any other nature, are made with respect to information concerning the product referred to in this document.  The information contained herein is, to the best of our knowledge and belief, accurate.  However, since the conditions of handling and use are beyond our control, it is impossible to foresee every health effect or exposure risk incurred by the use of this product.  All chemicals present unknown health hazards and should always be used with caution.  The information and recommendations contained herein are presented in good faith and are supplied pursuant to 29 CFR 1910.1200.  The user should review this information in conjunction with their knowledge of the application intended to determine the suitability of this product for such purpose.  In no event will the supplier be responsible for any damages of any nature whatsoever, resulting from the use, reliance upon, or the misuse of this information.  Furthermore, it is the direct responsibility of the user to comply with all applicable Federal, State and local regulations governing the use and disposal of this material.							Prepared by:  Regulatory Affairs Dept.