966069         MATERIAL SAFETY DATA SHEET

MEDICAL EMERGENCIES ONLY (24 Hour Service):   1-800-328-0026

Medical Calls from Outside of the USA:   1-651-222-5352

PROFESSIONAL PRODUCTS    ECOLAB INC.

370 WABASHA STREET NORTH

ST. PAUL MN 55102

Product Information:  1-800-247-5362        Issue Date: November 10, 1999

==========================================================================

1.0 IDENTIFICATION /

1.1 Product Name: SPRITZ

1.2  Product Type: Liquid Alkaline/Solvent Detergent

1.3  Hazard Rating:  Health:   1     Fire:   1     Reactivity:   0

--------------------------------------------------------------------------

Substances Subject to SARA 313 Reporting Are Indicated by "#"

--------------------------------------------------------------------------

2.0 HAZARDOUS COMPONENTS /                                    (mg/m3)

CAS No.     %    PEL     TWA

2.1  Sodium metasilicate                    6834-92-0   1-5     No      No

2.2  # 2-Butoxyethanol                       111-76-2     7    240     121

(ethylene glycol monobutyl ether) (absorbed through skin)

2.3  Nonionic surfactant                 confidential    1-5    No      No

--------------------------------------------------------------------------

PEL = OSHA 8 hour average in air         TWA = ACGIH 8 hour average in air

--------------------------------------------------------------------------

3.0 PHYSICAL DATA /

3.1  Appearance:  Clear pink liquid; solvent odor

3.2  Solubility in Water:  Mixes with water in all proportions

3.3  pH:   12.0 - 13.0  (100%)

3.4  Initial Boiling Point:  > 212 deg F

3.5  Specific Gravity:  1.100 @ 68 deg F

--------------------------------------------------------------------------

4.0 FIRE AND EXPLOSION DATA  /

4.1  Special Fire Hazards:  May flash, but does not support combustion.

4.2  Fire Fighting Methods:  Use method appropriate for surrounding fire.

--------------------------------------------------------------------------

5.0 REACTIVITY DATA /

5.1  Stability:  Stable under normal conditions of use and handling.

5.2  Conditions to Avoid:  High heat, ignition sources.  Mix only with

water.  Keep from freezing.

--------------------------------------------------------------------------

6.0 SPILL OR LEAK PROCEDURES /     USE PROPER PROTECTIVE EQUIPMENT

6.1  Cleanup:  Dike or dam large spills.  Pump to containers or soak up

on inert absorbent.  Flush residue to sanitary sewer.

6.2  Waste Disposal:  Consult state/local authorities for limits on

chemical waste disposal.

==========================================================================

7.0 HEALTH HAZARD DATA /                     CAUTION

7.1  Effects of Overexposure to Concentrate:

Eyes:  Causes minor to moderate irritation.

Skin:  May cause irritation, depending on degree of exposure.

If Swallowed:  May cause stomach distress, nausea or vomiting.

If Inhaled:  May cause breathing difficulties, dizziness, headache.

7.2  Note: In rats and mice, heavy doses of 2-butoxyethanol damage red

blood cells, which in turn can injure the liver, kidney and spleen.

These effects have not been observed in humans.

--------------------------------------------------------------------------

8.0 FIRST AID /

8.1  Eyes:  Flush with cool running water.  Remove contact lenses and

continue flushing for 15 minutes.

8.2  Skin:  Flush skin with water; wash with soap and water.  Remove

and wash contaminated clothing before reuse.

8.3  If Swallowed:  Rinse mouth; then drink 1 or 2 large glasses of

water.  DO NOT induce vomiting.  NEVER give anything by mouth to

an unconscious person.

8.4  If Inhaled:  If affected, move to fresh air.

IF IRRITATION OR DISCOMFORT PERSISTS, CALL A PHYSICIAN.

--------------------------------------------------------------------------

9.0 PROTECTIVE MEASURES /

9.1  FOR CONCENTRATE:

Eyes:  Wear safety glasses with side shields.  For severe exposure

wear chemical splash goggles or face shield.

Skin:  Wear any industrial rubber glove.

Respiratory: Ventilate to maintain air levels below exposure limits.

--------------------------------------------------------------------------

10.0 ADDITIONAL INFORMATION /

10.1 Purpose of 11/10/1999 issue:  Revise line 1.3, QC and editorial

update.  Replaces December 26, 1991 issue.

KEEP OUT OF REACH OF CHILDREN

The above information is believed to be correct with respect to the

formula used to manufacture the product.  As data, standards and

regulations change, and conditions of use and handling are beyond our

control, NO WARRANTY, EXPRESS OR IMPLIED, IS MADE AS TO THE

COMPLETENESS OR CONTINUING ACCURACY OF THIS INFORMATION.