|  | Spartan Chemical Company, Inc. Material Safety Data Sheet |
| --- | --- |

SECTION I: PRODUCT INFORMATION SECTION I: PRODUCT INFORMATION

| Product Name or Number (as it appears on label): | Product Division: |
| --- | --- |
| SHEEN 17 Product Number: 4017 | Janitorial |

| Spartan Chemical Company, Inc. | Product/Technical Information: | 1-(800)-537-8990 |
| --- | --- | --- |
| 1110 Spartan Drive | Medical Emergency: | 1-(888)-314-6171 (24 hours) |
| Maumee OH 43537 | Chemical Leak/Spil Emergency: | CHEMTREC 1-(800) 424-9300 (24 hours) |

| Shipping Description: | Buffing, polishing compounds, liquid, n.o.s. |
| --- | --- |

| NFPA Ratings: |  | HMIS Ratings: |  |
| --- | --- | --- | --- |
| Health: | 1- Slight | Health: | 1- Slight |
| Fire: | 0 - Minimal | Fire: | 0 - Minimal |
| Reactivity: | 0 - Minimal | Reactivity: | 0 - Minimal |
|  |  | Pers. Prot. Equip.: | B - Safety Glasses; gloves |

SECTION II: HAZARDOUS INGREDIENTS 
SECTION II: HAZARDOUS INGREDIENTS
(Listed when present at 1% or greater, carcinogens at 0.1% or greater) All component chemicals are listed or exempted from listing on the "TSCA Inventory" of chemical substances maintained by the U.S. Environmental Protection Agency.

| Chemical Name(s) | % W | CAS Registry No. | Table Z- 1 - A |  |  | NTP, IARC or OSHA Carcinogen |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TWA mg/m3 | STEL mg/m 3 | CELING mg/m3 |  |
| *Diethylene glycol monoethyl ether | 4.5 | 111-90-0 | Not Established | Not Established | Not Established | No |
| *Dibutyl phthalate | 1.2 | 84-74-2 | 5 | Not Established | Not Established | No |

Comment: *This substance is subject to the reporting requirements of Section 313 of the Emergency Planning and Community Right-To-Know Act of 1986 (SARA) and of 40 CFR 372.

SECTION III: PHYSICAL DATA 
SECTION III: PHYSICAL DATA

| Boiling Point: | 210-215 °F | Vapor Pressure: | 18 mm Hg @ 75°F |
| --- | --- | --- | --- |
| Vapor Density (AIR = 1): | Unknown | Solubility in Water: | Complete |
| pH: | 8.0-8.5 | Specific Gravity (H2O=1): | 1.025 |
| Evaporation Rate (but.ace.=1): | <1 | Percent Solid by Weight: | 16-18 |
| Physical State: | Liquid |  |  |

| Appearance & Odor: | Translucent emulsion, slight ammoniacal odor |
| --- | --- |

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA 
SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

| Flash Point: | None | Method Used: | ASTM-D56 |
| --- | --- | --- | --- |
| Flammable Limits: | N/A | Flame Extension: | N/A |

| Extinguishing Media: | N/A |
| --- | --- |
| Special Fire Fighting Procedures: | N/A |
| Unusual Fire & Explosive Hazards: | N/A |

SECTION V: HEALTH HAZARD DATA 
SECTION V: HEALTH HAZARD DATA

| Threshold Limit Value: | Not Established | Primary Routes of Entry: | Inhalation |
| --- | --- | --- | --- |

| Effects of Overexposure- Conditions to Avoid: | Vapors may be irritating to eyes and respiratory tract and may produce symptoms of headache and nausea in poorly ventilated areas. |
| --- | --- |
| Conditions Aggravated by Use: | Unknown |

Emergency & First Aid Procedures:

| Eyes: | In case of contact; immediately flush eyes with water for at least 15 minutes. Remove contact lenses. Call a physician. |
| --- | --- |
| Skin: | In case of contact; flush with water. |
| Ingestion: | If swallowed; drink large quantities of water; call a physician. |

SECTION VI: REACTIVITY DATA 
SECTION VI: REACTIVITY DATA

| Stability: | Stable | Incompatible Materials: | None known |
| --- | --- | --- | --- |
| Hazardous Decomposition Products: | None known | Hazardous Polymerization: | Will Not Occur |

SECTION VII: SPILL OR LEAK PROCEDURES 
SECTION VII: SPILL OR LEAK PROCEDURES

| Steps to be Taken in Case Material is Released or Spilled: | Small spills, flush with water to sewer system. Dike and contain large spill with inert material and transfer liquid to containers for recovery or disposal. |
| --- | --- |
| Waste Disposal Method: | Incineration or approved landfill. |

SECTION VIII: SPECIAL PROTECTION INFORMATION 
SECTION VIII: SPECIAL PROTECTION INFORMATION

| Respiratory Protection: | Not normally required |
| --- | --- |
| Ventilation: | Good general ventilation should be sufficient for most conditions. Local exhaust ventilation may be necessary for some operations. |
| Protective Gloves(Specify Type): | Impervious gloves |
| Eye Protection(Specify Type): | Safety glasses |
| Other Protective Equipment: | None required |

SECTION IX: SPECIAL PRECAUTIONS 
SECTION IX: SPECIAL PRECAUTIONS

| Precautions; Handling & Storing: | Keep from freezing. |
| --- | --- |
| Other Precautions: | None |

| © SCC 09/28/1999 | Name: | Ronald T. Cook | Title: | Manager, Regulatory Affairs |
| --- | --- | --- | --- | --- |
| SHEEN 17 | Effective Date: | 09/28/1999 | Supercedes: | 02/11/1998 |
| Ref: 29 CFR 1910.1200 (OSHA) | Changes: | Address and Format |  |  |

This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained herein. Actual conditions of use and handling are beyond sellers control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State and Local laws and regulations.