# SAFETY DATA SHEET

---------------------------------------------------------------------------------------------------------------------------------

SECTION I - IDENTIFICATION

DATE: September 5, 2024

CONTINENTAL CHEMICAL CO.	EMERGENCY PHONE

4920 E. WASHINGTON BLVD.		TRANSPORTATION: CHEMTREC: 800-424-9300

COMMERCE CA. 90040 			NON-TRANSPORTATION: HJB INC: 323-268-1288

PRODUCT NAME:    NO RINSE QUAT

MANUFACTURER:	CONTINENTAL CHEMICAL

HMIS: 210 C

# SECTION II- HAZARDS IDENTIFICATION

CLASSIFICATION

GHS CLASSIFICATION:			SKIN CORROSION (CATEGORY 3)

SERIOUS EYE DAMAGE (CATEGORY 2B)

ACUTE AQUATIC TOXICITY (CATEGORY 3)

GHS LABEL ELEMENTS

PICTOGRAM:

SIGNAL WORD:			DANGER

HAZARD STATEMENT:	H333: MAY BE HARMFUL IF SWALLOWED AND ENTERS AIRWAYS

H402: HARMFUL TO AQUATIC LIFE

PRECAUTIONARY STATEMENTS:

HMIS CLASSIFICATION:	HEALTH HAZARD:	2

FLAMMABILITY:	1

PHYSICAL HAZARDS:	0

NFPA RATING:	HEALTH HAZARD:	4

FIRE:			1

REACTIVITY HAZARD:	0

POTENTIAL HEALTH EFFECTS

INHALATION:	NO EMERGENCY CARE IS ANTICIPATED.

SKIN:	NO EMERGENCY CARE IS ANTICIPATED.

EYES:	FLUSH EYES THOROUGHLY WITH WATER FOR SEVERAL MINUTES.

INGESTION:	TOXIC IF SWALLOWED. DO NOT INGEST.

SECTION III CHEMICAL COMPOSITION

COMPONENT						CAS#		CONCENTRATION

ALKYL DIMETHYL BENZYL AMMONIUM CHLORIDE	 68424-85-1	  	5.0

ALKYL DIMETHYL ETHYLBENZYL AMMONIUM CHLORIDE	 85409-23-0                	5.0

# SECTION IV FIRST AID MEASURES

IF INHALED:                                                 IF INHALED, MOVE PERSON INTO FRESH AIR. IF NOT

BREATHING, GIVE ARTIFICIAL RESPIRATION.

CONSULT A PHYSICIAN.

SKIN CONTACT:                                          TAKE OFF CONTAMINATED CLOTHING AND SHOES

IMMEDIATELY. FLUSH SKIN WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES. CONSULT A PHYSICIAN.

EYE CONTACT:	FLUSH EYES WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES AND CONSULT A PHYSICIAN.

IF INGESTED:	DO NOTINDUCE VOMITING UNLESS INSTRUCTED TO DO SO BY A PHYSICIAN. NEVER GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON. RINSE MOUTH WITH WATER. CONSULT A PHYSICIAN.

SECTION V FIREFIGHTING MEASURES

FLAMMABILITY OVERVIEW:	DO NOT USE WATER JET.

FLASH POINT:	> 212°F ASTM D93

EXTINGUISHING MEDIA:	USE DRY CHEMICAL, CARBON DIOXIDE, WATER FOG, FOAM, SAND OR EARTH.

SPECIAL PROTECTIVE EQUIPMENT:	WEAR A SELF-CONTAINED BREATHING APPARATUS (SCBA) FOR FIREFIGHTING, IF NECESSARY.

HAZARDOUS COMBUSTION PRODUCTS: HAZARDOUS DECOMPOSITION PRODUCTS UNDER FIRE CONDITION: OXIDES OF CARBON.

SECTION VI ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS:	USE PERSONAL PROTECTIVE EQUIPMENT. AVOID BREATHING VAPORS, MIST OR GAS. ALWAYS ENSURE ADEQUATE VENTILATION. EVACUATE PERSONNEL TO SAFE AREAS.

ENVIRONMENTAL PRECAUTIONS:	PREVENT FURTHER LEAKAGE OR SPILLIAGE. PREVENT FROM ENTERING DRAINS AND WATERWAYS. DISCHARGE INTO THE ENVIRONMENT MUST BE AVOIDED.

CONTAINMENT AND CLEAN UP:	SOAK UP WITH INERT ABSORBENT MATERIAL AND DISPOSE OF AS HAZARDOUS WASTE. KEEP IN SUITABLE, CLOSED CONTAINERS FOR DISPOSAL.

# SECTION VII HANDLING AND STORAGE

SAFE HANDLING:	USE PERSONAL PROTECTIVE EQUIPMENT. AVOID BREATHING VAPORS, MIST OR GAS. ALWAYS ENSURE ADEQUATE VENTILATION.

SAFE STORAGE:	KEEP CONTAINERS TIGHTLY CLOSE IN A DRY WELL-VENTILATED PLACE. CONTAINERS WHICH ARE OPENED MUST BE CAREFULLY RESEALED AND KEPT UPRIGHT TO PREVENT LEAKAGE. STORE AWAY FROM INCOMPATIBLE MATERIALS.

# SECTION VIII EXPOSURE CONTROLS/ PERSONAL PROTECTION

EXPOSURE LIMITS

COMPONENT			CAS NUMBER	EXPOSURE LIMIT		BASIS

ALKYL DIMETHYL BENZYL-		68424-85-1		NE		TLV

AMMONIUM CHLORIDE

PERSONAL PROTECTIVE EQUIPMENT

EYE PROTECTION:	WEAR TIGHTLY FITTING SAFETY GOGGLES AND FACE SHIELD. USE EQUIPMENT FOR EYE PROTECTION TESTED AND APPROVED UNDER NIOSH. HAVE EYE-WASH STATIONS AVAILABLE WHERE EYE CONTACT CAN OCCUR.

HAND PROTECTION:	HANDLE WITH CHEMICAL-RESISTANT GLOVES. GLOVES MUST BE INSPECTED PRIOR TO USE. DISPOSE OF CONTAMINATED GLOVES. WASH AND DRY HANDS AFTER USE.

SKIN PROTECTION:	WEAR COMPLETE SUIT PROTECTION AGAINST CHEMICALS, INCLUDING CHEMICAL-RESISTANT BOOTS. SAFETY SHOWERS SHOULD BE LOCATED IN THE WORK AREA WHERE SKIN CONTACT CAN OCCUR.

RESPIRATORY PROTECTION:	WHERE RISK ASSESSMENT SHOWS AIR-PURIFYING RESPIRATORS ARE APPROPRIATE, USE A NIOSH-APPROVED FULL FACE RESPIRATOR WITH APPROPRIATE CARTRIDGES. RESPIRATORY PROTECTION MUST BE PROVIDED IN ACCORDANCE WITH OSHA 29 CFR 1910.134.

GENERAL CONTROLS:	ALWAYS ENSURE ADEQUATE VENTILATION AND THAT WORKING AREAS CONTAINS SAFETY SHOWERS AND EYE WASH STATIONS. HANDLE MATERIAL IN ACCORDANCE WITH GOOD INDUSTRIAL HYGIENE AND SAFETY PRACTICES.

SECTION IX PHYSICAL AND CHEMICAL PROPERTIES

PHYSICAL STATE:	LIQUID

COLOR:	CLEAR

ODOR:	TYPICAL

pH (as is):	6.0 – 8.0

VAPOR PRESSURE:	NOT ESTABLISHED

BOILING POINT:	212° - 220°F

VAPOR DENSITY (air = 1):	N/A

SPECIFIC GRAVITY:	1.002 – 1.003 @ 25°C

SOLUBILITY IN WATER:	COMPLETE

SECTION X STABILITY AND REACTIVITY

CHEMICAL STABILITY:	STABLE UNDER RECOMMENDED STORAGE CONDITIONS.

MATERIALS TO AVOID:	EXTREME TEMPERATURE AND DIRECT SUNLIGHT. STRONG OXIDIZING AGENTS, ACIDS

HAZARDOUS DECOMPOSITION:	OXIDES OF CARBON AND SOOT.

SECTION XI TOXICOLOGICAL INFORMATION

ACUTE TOXICITY

COMPONENT			CAS NUMBER	TEST			TOXICITY

ALKYL DIMETHYL BENZYL-		68424-85-1	ORAL-LD50 (Rat)		344 mg/Kg

AMMONIUM CHLORIDE				DERMAL-LD50 (Rabbit)	3340 mg/Kg

POTENTIAL HEALTH EFFECTS

INHALATION:	MAY BE HARMFUL IF INHALED.

SKIN:	BRIEF CONTACT MAY CAUSE SLIGHT SKIN IRRITATION WITH LOCAL REDNESS

EYES:	MAY CAUSE SEVERE EYE IRRITATION

INGESTION:	HARMFUL IF SWALLOWED. DO NOT INGEST.

SYMPTOMS OF EXPOSURE:	NONE

CHRONIC EFFECTS OF EXPOSURE:	NO DATA AVAILABLE

CARCINOGENICITY:	NO COMPONENTS OF THIS PRODUCT AT LEVELS GREATER THAN 0.1% IS IDENTIFIED AS CARCINOGENIC BY IARC, ACGIH, OR OSHA.

SECTION XII ECOLOGICAL INFORMATION

ACUTE ECOTOXICITY

COMPONENT			CAS NUMBER	ORGANISM		ECOTOXICITY

ALKYL DIMETHYL BENZYL-		68424-85-1	FATHEAD MINNOW	LC50, 0.28 mg/L 96 hr.

AMMONIUM CHLORIDE

ECOLOGICAL EFFECTS

PERSISTENCE AND DEGRADABILITY:	NO DATA AVAILABLE

BIOACCUMULATION POTENTIAL:	NOT EXPECTED TO BIO-ACCUMULATE

MOBILITY IN SOIL:			NO DATA AVAILABLE

OTHER ADVERSE EFFECTS:	NONE

SECTION XIII DISPOSAL

DISPOSAL:	SURPLUS AND NON-RECYCLABLE MATERIAL SHOULD BE TREATED AS HAZARDOUS WASTE AND BE DISPOSED OF BY A LICENSED DISPOSAL COMPANY. DO NOT RELEASE INTO SEWERS OR WATERWAYS.

CONTAMINATED PACKAGING:	DISPOSE OF AS UNUSED PRODUCT.

SECTION XIV TRANSPORTATION

PRODUCT NAME:			NO RINSE QUAT

PROPER SHIPPING NAME:	CORROSIVE LIQUID, N.O.S. (QUATERNARY    AMMONIUM CHLORIDE)

ID NUMBER:				UN 1760

HAZARD CLASS OR DIVISION:	8

PACKING GROUP:			III

DOT HAZARDOUS SUBSTANCE:

SECTION XV REGULATORY INFORMATION

US FEDERAL

SARA 302 COMPONENTS:		NONE LISTED

SARA 311/312 HAZARDS:		ACUTE HEALTH HAZARD

SARA 313 COMPONENTS:		NONE LISTED

TSCA INVENTORY:

EUROPEAN UNION

EC INVENTORY:

STATE REGULATIONS

CA PROP 65:				NONE LISTED

SECTION XVI OTHER INFORMATION

SDS VERSION:				2.0

REVISION DATE:			09/03/2020

SUPERSEDES:				UN ID NUMBER

THE MANUFACTURER BASED ON ITS OWN DATA AND THE WORK OF OTHERS HAS ASSEMBLED THE INFORMATION IN THIS DATA SHEET. THE MANUFACTUER MAKES NO WARRANTIES, EXPRESS OR IMPLIED, AS TO THE ACCURACY, COMPLETENESS, OR ADEQUACY OF THE INFORMATION CONTAINED HEREIN. THE MANUFACTURER SHALL NOT BE LIABLE (REGARDLESS OF FAULT) TO VENDEE, THE VENDEE’S EMPLOYEES, OR ANYONE FOR ANY DIRECT, SPECIAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE ACCURACY, COMPLETENESS, ADEQUACY, OR FURNISHING OF SUCH INFORMATION.